# FWPCOT2209 Produce finger jointed timber

Release: 1

FWPCOT2209 Produce finger jointed timber

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT2209B Produce finger jointed timber, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to produce finger jointed timber, including preparing and mixing glue and maintaining production processes to ensure quality and continuity of supply.

The unit applies to timber products operators involved with the production of finger jointed timber in a forest products factory setting

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Timber Manufactured Products

Sawmilling and Processing

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| 1. Prepare for operation | 1.1 Review work order and where required check with appropriate personnel. 1.2 Identify glue requirements and mix to ensure minimal wastage; dispose of excess material in line with environmental protection practices. 1.3 Select appropriate equipment and check for operational effectiveness in line with manufacturer’s recommendations. |
| 2. Start jointing process | 2.1 Prepare materials and check samples in line with organisational procedures. 2.2 Use equipment in line with organisational safety procedures, manufacturer’s instructions environmental protection practices. 2.3 Commence jointing process, identify problems and adjust settings. 2.4 Check finished joints for strength and appearance in line with organisational quality standards. 2.5 Dispose of sub-standard samples and finished products in line with environmental protection practices. |
| 3. Maintain production flow | 3.1 Run automatic jointing process and regularly check correct operation of all sections. 3.2 Monitor processes to ensure continuity of supply and processing. 3.3 Evaluate product quality regularly and make required adjustments. 3.4 Clear work area regularly in line with organisational safety procedures. 3.5 Identify routine processing and equipment faults and resolve or report to appropriate personnel. 3.6 Complete production and quality records and report in line with organisational procedures. |
| 4. Complete operator maintenance | 4.1 Follow organisational safety procedures to lock out equipment. 4.2 Check cutters and saw blades for bluntness or damage. 4.3 Remove and replace cutters and blades in line with manufacturer’s recommendations. 4.4 Dispose of used blades in line with organisational procedures and environmental protection practices. 4.5 Record and report equipment faults and maintenance requirements to appropriate personnel. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | calculate quantity of glue to meet work order production requirements calculate routine mixing ratios for glue calculate time for glue to cure complete routine production records involving quantities tallied against orders. |
| Oral communication skills to: | ask questions and actively listen to clarify contents of work orders make accurate verbal reports on processing and equipment faults. |
| Reading skills to: | interpret: workplace health and safety and other organisational procedures work orders safety data sheets for glue manufacturers’ instructions and maintenance recommendations. |
| Writing skills to: | complete accurate basic records for production outcomes, equipment faults and maintenance requirements. |
| Planning and organising skills to: | efficiently and logically sequence the stages of preparation and production using work order to guide activities. |
| Problem-solving skills to: | identify and dispose of sub-standard samples and finished products recognise faulty equipment and resolve by reporting or rectifying. |
| Technology skills to: | select, use and maintain appropriate machinery, tools and personal protective equipment used for producing finger jointed timber. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPICOT2209B Produce finger jointed timber

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT2209 Produce finger jointed timber

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT2209B Produce finger jointed timber, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

produce finger jointed timber in line with an individual work orders involving the following processes:

identify and confirm cross section of cleats/shooks to match finger jointing machine setup

load cleats/shooks to conveyor

reject cleats/shooks not meeting grading requirements

feed cleats/shooks to jointing operation

machine fingers on cleats/shooks

cure and apply glue to cleats/shooks

monitor process to ensure continuity of production by completing these processes:

recognise and rectify blunt or damaged cutters

check and adjust settings for glue curing

clean and fill glue spreader

coordinate supply of cleats/shooks

dock finished boards with others to ensure continuous supply.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

safely disposing of, recycling and reusing timber and other waste

types of glue used for producing finger jointed timber; procedures and standards for mixing, curing and applying it

range of industry standard cross sections, their strength, tolerances and applicable terminology

types of tools and equipment used for producing finger jointed timber; their purpose, features, operation and basic non-specialist repair and maintenance:

that used for glue mixing and preparation

finger jointing machine stations and equipment

conveyors

methods for assessing cutters and blade condition

monitoring processes for automated jointing machines

organisational procedures specific to producing finger jointed timber:

workplace health and safety with particular emphasis on equipment lock out

use and maintenance of equipment

communication reporting lines

recording and reporting production outcomes and equipment faults.

# Assessment Conditions

The following resources must be made available:

tools and equipment used to produce finger jointed material

maintenance tools and equipment for saws

glue

consumable components; cutters, saw blades

personal protective equipment suitable for producing finger jointed timber

a diverse, comprehensive and commercial product range of timber

manufacturers:

instructions for mixing and curing glue

instructions for equipment use

recommendations for cleaning, repairing and maintaining

work order with specific instructions for producing finger jointed timber products

template documents for recording production outcomes and equipment faults

organisational procedures for producing finger jointed timber.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47