# FWPCOT2226 Debark logs mechanically

Release: 2

FWPCOT2226 Debark logs mechanically

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.2. |

# Application

This unit of competency describes the skills and knowledge required to debark logs mechanically to a set diameter and length in a saw mill or wood processing plant.

The unit applies to individuals who debark logs using mechanical equipment. Operators generally work under broad direction and take responsibility for their work.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Core Technical (COT)

# Elements and Performance Criteria

| ELEMENTS | PERFORMANCE CRITERIA |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| 1. Prepare for debarking | 1.1 Review work order and clarify with appropriate personnel 1.2 Assess type and quantity of logs to be processed by mechanical debarker 1.3 Select equipment and check for operational effectiveness 1.4 Plan debarking process in line with site work, health and safety and operating procedures and environmental requirements 1.5 Identify emergency plan procedures to follow in the event of an emergency 1.6 Establish and maintain communication with others in line with work health and safety requirements |
| 2. Debark logs | 2.1 Carry out pre-operational checks on mechanical debarker in line with workplace procedures or manufacturers’ recommendations 2.2 Feed material into the machine at the rate prescribed by workplace requirements 2.3 Regularly check and adjust conveyors to maximise feed rate and maintain desired quality and output 2.4 Segregate logs with defects and dispose of in line with site procedures and environmental requirements 2.5 Direct debarked logs to storage in a designated location in line with work order 2.6 Report processing faults to appropriate personnel |
| 3. Conduct operator maintenance | 3.1 Lock-out mechanical debarker in line with site procedures 3.2 Conduct visual inspection to identify issues and faults 3.3 Check mechanical debarker for blunt of damaged cutters and remove and replace, in line with manufacturers’ specifications 3.4 Repair or replace blunt or damaged cutters in line with site procedures and environmental requirements 3.5 Record and report mechanical debarker operations, faults and maintenance activities |

# Foundation Skills

This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | read standard operating procedures and/or manufacturers’ instructions for the operation of debarker |
| Writing | record operation, production and machine maintenance on paper based or electronic media |
| Oral Communication | interact verbally with others on site to ensure safe and effective debarking operations use appropriate vocabulary, including technical language directly relevant to role |
| Numeracy | interpret gauges relevant to safe operation of debarker |
| Navigate the world of work | understand main tasks, responsibilities and boundaries of own role |
| Interact with others | communicate and report operational and safety information to on-site personnel |
| Get the work done | recognise and respond to routine problems plan and organise own work activities and resources |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT2226 Debark logs mechanically Release 2 | FWPCOT2226 Debark logs mechanically Release 1 | No significant changes were applied to this unit. One new PC1.5 added and PC1.4 edited to align with existing KE. Minor additions to the foundation skills. | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47