# FWPCOT2239 Trim and cut felled trees

Release: 3

FWPCOT2239 Trim and cut felled trees

# Modification History

| Release | Comments |
| --- | --- |
| Release 3 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Application

This unit describes the outcomes required to trim and cut felled trees with a chainsaw and to complete operator maintenance. It applies to situations where the production of timber is not the primary focus of the activity.

The unit applies to a forestry worker, chainsaw operator, forest harvester, harvesting technician and arboriculture worker.

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for trimming and cutting | 1.1 Review work order and where required check with appropriate personnel 1.2 Select appropriate personal protective and other equipment and check for operational effectiveness 1.3 Plan trimming and cutting activities in line with work order and environmental conditions |
| 2. Visually assess felled trees | 2.1 Assess tree location and stability for conditions likely to affect safety of trimming and cutting activities 2.2 Visually assess tree for defects and identify stresses within tree 2.3 Mark trees too dangerous to cut safely and refer to appropriate personnel |
| 3. Plan cutting | 3.1 Move or stabilise tree for safe cutting in line with site procedures 3.2 Select cutting pattern to optimise time and manageable removal of sections 3.3 Identify options for utilisation of product 3.4 Plan cutting sequence to maintain control of cut sections and minimise cutting problems 3.5 Select cutting positions considering tree stresses 3.6 Clear debris from work area to allow safe access and prevent saw damage and personal injury |
| 4. Use chainsaw to trim and cut trees | 4.1 Monitor location and movement of other personnel and modify work to ensure safety 4.2 Establish and maintain communication to ensure safety 4.3 Secure tree section on each side of planned cut; evaluate and control potential movement 4.4 Operate chainsaw to cut tree and limbs and adjust cutting technique in response to movement and condition of tree and limbs 4.5 Prepare cut sections for removal from the site |
| 5. Complete equipment maintenance | 5.1 Follow organisational safety procedures to lock out equipment 5.2 Check chain for bluntness or damage 5.3 Remove, sharpen, adjust and replace chain and other components in line with manufacturer recommendations 5.4 Record and report trimming and cutting records, equipment faults and maintenance requirements to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Numeracy | Plan time-efficient cutting patterns and sequences Complete routine calculations and report on size, length and diameter of cut trees |
| Oral communication | Ask open and closed questions and actively listen to clarify contents of work plans |
| Reading | Interpret: workplace health and safety and other organisational procedures work orders manufacturers maintenance recommendations |
| Writing | Complete accurate basic records for tree trimming and cutting processes, equipment faults and maintenance requirements |
| Get the work done | Efficiently and logically sequence the stages of preparation for and cutting of trees using work order to guide activities |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT2239 Trim and cut felled trees Release 3 | FWPCOT2239 Trim and cut felled trees Release 2 | Minor edits to achieve consistency with wording in another FWPHAR unit; and enable flexibility of the unit application | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT2239 Trim and cut felled trees

# Modification History

| Release | Comments |
| --- | --- |
| Release 3 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.

There must be evidence that, on at least one occasion, the individual has:

safely and efficiently used a chainsaw to complete these activities:

trim and cut hardwood or softwood trees

delimb trees

trim branches

trim burls and other small growths

used the following cutting techniques:

bridging cuts

swinging cuts

side bind cuts

boring

wedge cut

ripping cuts

trimmed and cut trees in varied weather conditions

completed the following maintenance activities on a chainsaw:

sharpening

refuelling

inspection, identification and replacement of bar, chain, drive sprockets

bar oil checks and top up

air filter, check, clean and replacement

cleaning of saw body, air intake vents and cooling fans

fitting spare chain and tension chain.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

environmental protection measures including:

action to limit the impact of noise, wind speed and direction, fallen trees, ground slope, ground hazards and obstacles, general forest lean

action to limit the impact to ground growth, canopy, density of trees

action to limit the impact on flora, fauna and habitat

soil and water protection

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste

key features of these varying site conditions and effects on tree cutting and trimming activities:

ground growth

ground hazards and obstacles

ground slope

key features of these varying weather conditions and effects on tree cutting and trimming activities:

dry, wet and adverse weather conditions

low and moderate wind speed and direction

typical tree defects and how they affect tree cutting activities:

splits

falling damage

fire damage

insect infestation

pipe

shake

twist

knots and resin pockets

the presence of foreign bodies such as stakes and nails

typical tree stresses and how they affect tree cutting activities:

tension (fibres being stretched)

compression (fibres being squashed)

neutral fibres

range of chainsaw cutting techniques:

bridging cuts

swinging cuts

side bind cuts

delimbing

boring

wedge cut

ripping cuts

cutting patterns used to optimise time and recovery of product from trees

cutting sequences used to maintain control of cut sections and minimise cutting problems

purpose, features, operation and basic non-specialist repair and maintenance of chainsaws

methods for assessing chain condition

organisational procedures specific to tree trimming and cutting activities:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting tree cutting outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting

resources, equipment and materials:

chainsaws

chainsaw fuel and oil containers

support tools appropriate for work tasks (hammer, wedges, logging tape, cant hook and/or breaking bar)

mobile plant and equipment appropriate for work tasks (wood chipper and/or loaders)

maintenance tools and equipment for chainsaws

personal protective equipment suitable for cutting trees and maintaining chainsaws

specifications:

manufacturer’s instructions for use and maintenance of equipment

work order with specific instructions for tree cutting activities

template reports for recording tree cutting outcomes, equipment faults and maintenance requirements

organisational procedures for tree trimming and cutting activities.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47