# Assessment Requirements for FWPCOT2244 Operate and maintain a thicknesser

Release: 1

Assessment Requirements for FWPCOT2244 Operate and maintain a thicknesser

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has:

set up, adjusted and operated a thicknesser to dress timber according to required dimensions, cutting sequence, feed rates and finish

in doing the above job, the individual has:

reviewed work order to determine job requirements

followed work order and relevant workplace health and safety and environmental protection procedures

selected equipment and materials according to job requirements

planned the sequence of machine operations

identified and disposed of defective timber before processing

complied with workplace health and safety procedures relevant to dressing timber

assessed dressing conditions

adjusted conditions to optimise feed rate and maintain finished dimensions

completed basic routine equipment maintenance

completed production and maintenance reports.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

typical timber defects and potential impact on the quality of dressed timber product:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

typical dressing problems triggering adjustment of process to ensure product quality:

blunt or damaged cutters

burn marks

extra cuts

excessive cutter marks

dimensional errors

industry standards and terminology for timber profiles, lengths, cross-sections and tolerances

types, purpose, features, operational procedures, components and basic maintenance requirements of thicknessers

problems that can occur when dressing timber:

equipment faults and malfunctions

quality of product and machine settings

efficient rejection of defective timber before dressing

transfer and flow of material

dressing problems

feed rate

product finish

methods for assessing cutters condition

types of risk and hazards and mitigation measures associated with dressing timber

workplace procedures specific to dressing timber:

workplace health and safety requirements, with particular emphasis on equipment lock-out, use of personal protective equipment (PPE) and safe manual handling

selection, use and maintenance of equipment

communication reporting lines

recording and reporting production outcomes and equipment faults

environmental protection practices for timber processing or manufacturing facilities:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing or manufacturing facility or an environment that accurately reflects workplace conditions

resources, equipment and materials:

thicknesser

maintenance tools and equipment

consumable components of thicknesser

PPE suitable for dressing timber

a diverse, comprehensive and commercial range of timber

specifications:

template documents for recording production outcomes and equipment faults

manufacturer specifications and instructions for thicknesser

work order detailing material to be dressed, special client requirements and production timelines

written workplace procedures for operating and maintaining thicknesser.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47