# FWPCOT2256 Trim and cut felled trees

Release: 2

FWPCOT2256 Trim and cut felled trees

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 6.3. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Application

This unit describes the skills and knowledge required to trim and cut felled trees with a chainsaw and complete operator maintenance.

The unit applies to individuals who trim and cut felled trees as part of arboriculture, forestry, agriculture, conservation and land management, local government, emergency services and other government agency operations. With the exception of minor forest produce, this unit does not apply to commercial harvesting operations.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation, standards and industry codes of practice that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for trimming and cutting felled trees | 1.1 Determine job requirements from work order or instruction and, where required, seek clarification from appropriate personnel 1.2 Confirm safety and environmental requirements for the task according to workplace procedures 1.3 Identify, assess and take actions to mitigate risks and hazards associated with trimming and cutting felled trees 1.4 Plan trimming and cutting activities according to work order or instruction and environmental conditions 1.5 Consult and maintain communication with team members and other appropriate personnel to ensure that work is coordinated effectively with others in the workplace 1.6 Obtain tools and equipment needed for the work, and check for correct operation and safety 1.7 Select, fit and use personal protective equipment |
| 2. Assess felled trees visually | 2.1 Inspect site for conditions likely to affect safe implementation of trimming and cutting activities 2.2 Identify and report site environmental conditions to supervisor 2.3 Assess felled tree visually to identify defects and stresses within felled tree 2.4 Mark felled trees too dangerous to cut safely, and refer to appropriate personnel |
| 3. Plan cutting felled trees | 3.1 Move or stabilise felled tree for safe cutting according to workplace procedures 3.2 Select cutting pattern to optimise time and ensure efficient removal of sections 3.3 Identify options for utilisation of product 3.4 Plan cutting sequence to maintain control of cut sections and minimise cutting problems 3.5 Select cutting positions, considering felled tree stresses 3.6 Clear debris from work area to allow safe access and prevent saw damage and personal injury |
| 4. Use chainsaw to trim and cut felled trees | 4.1 Monitor location and movement of other personnel, and modify work to ensure safety 4.2 Establish and maintain communication with team members to ensure safety 4.3 Secure felled tree section on each side of planned cut as required, and evaluate and control potential movement 4.4 Operate chainsaw to cut felled tree and limbs, and adjust cutting technique in response to movement and condition of felled tree and limbs 4.5 Apply retrieval techniques for stuck, pinched or jammed part of chainsaws 4.6 Prepare cut sections for removal from site |
| 5. Complete equipment maintenance | 5.1 Follow workplace safety procedures and manufacturer instructions to lock out equipment 5.2 Check chain for bluntness or damage 5.3 Remove, sharpen, adjust or replace chain and other components according to manufacturer recommendations 5.4 Report trimming and cutting records, equipment faults and maintenance requirements to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace documentation to determine requirements |
| Writing | Use technical and workplace specific vocabulary to accurately and legibly complete workplace records and forms |
| Oral communication | Ask questions and actively listen to clarify contents of work plans |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT2256 Trim and cut felled trees Release 2 | FWPCOT2239 Trim and cut felled trees Release 1 | Minor change to Performance Evidence to correct an editorial error | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT2256 Trim and cut felled trees

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 6.3. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

assessed, planned and conducted the trimming and cutting on six felled trees using a chainsaw and demonstrated three or more cut types from the list below:

bridging cuts

swinging cuts

side bind cuts

boring cuts

wedge cuts

ripping cuts

step cuts.

In performing this work, there must be evidence that the individual has:

followed workplace policies and procedures, current workplace health and safety legislation, regulations and related industry standards and codes of practice applicable to trimming and cutting felled tree operations

checked and sharpened or changed a saw chain post trimming and cutting operation on one occasion according to manufacturer instructions.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

environmental protection measures suitable for a worksite where trimming and cutting operations are performed:

noise control

protection of flora, fauna and habitat

protection of soil

protection of water

disposal, recycling and reuse of waste

cleaning of plant, equipment and tools

protection of adjacent land uses and fixed assets

vehicle control

access restrictions

fire prevention

protection of cultural heritage

hazard control measures suitable for a worksite where trimming and cutting operations are performed, related to:

trees and other vegetation

ground conditions

noise

fire and other emergency situations

vehicle and traffic access

foot access

plant, tools and equipment

weather conditions

adjacent land uses

dangerous goods

tree defects and how they affect tree cutting activities:

splits

falling damage

fire damage

insect infestation

pipe

shake

twist

knots and resin pockets

the presence of foreign bodies such as stakes and nails

felled tree stresses and how they affect tree cutting activities:

tension (fibres being stretched)

compression (fibres being squashed)

neutral fibres

chainsaw cutting techniques:

bridging cut

swinging cut

side bind cut

boring cut

wedge cut

ripping cut

step cut

cutting patterns used to optimise time and recovery of product from felled trees

cutting sequences used to maintain control of cut sections and minimise cutting problems

purpose and procedures for routine operator maintenance of chainsaws

procedures used to free a pinched or stuck chainsaw

methods for assessing chain condition

workplace procedures specific to felled tree trimming and cutting activities:

workplace health and safety, with particular emphasis on equipment lock-out and use of personal protective equipment (PPE)

communication reporting lines

reporting felled tree cutting outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting

resources, equipment and materials:

chainsaw and accessories

suitable felled trees to trim and cut

PPE required in trimming and cutting felled trees

communication system

specifications:

access to workplace safety and environmental policies and procedures applicable to trimming and cutting operations and chainsaw use

access to workplace standard operating procedures or operator manual for chainsaws.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47