# Assessment Requirements for FWPCOT3221 Rehabilitate tracks, quarries and landings

Release: 1

Assessment Requirements for FWPCOT3221 Rehabilitate tracks, quarries and landings

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT3221B Rehabilitate tracks, quarries and landings, which was first released with FPI11 Forest and Forest Products Training Package Version 1. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

plan to safely rehabilitate tracks, quarries and landings after harvesting operations that includes inspection of:

analysing the current state of the track, quarry or landing

identifying natural water flow and water courses

checking for recoverable logs and equipment

considering the impact of harvesting or transportation.

and which includes consideration of:

ground slope and stability

damage to the environment

weather conditions

access to suitable equipment and other resources

safety

further damage that may result from renovations

waste disposal.

safely and efficiently use one or more of the following items of equipment and tools appropriate to the task and according to manufacturer’s instructions

dozer

front-end loader

skidder

grader using bucket or blade attachments

perform rehabilitation tasks that returns the environment to its original or near to original condition

perform renovation which includes where required:

pulling out and removing construction materials

breaking up ramps and constructions

filling and ripping surfaces

breaking up and burying crushed rock, sand and other stabilising materials

spreading and compacting top soil over base materials

establish run-off and drainage channels suitable to the site

conduct revegetation with plants suited to the location

manage the problems and risks associated with performing rehabilitation tasks.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

workplace requirements for performing the rehabilitation of tracks, quarries and landings

environmental protection requirements, including the safe disposal of waste material

problem identification and resolution strategies, and common fault finding techniques

tools and equipment used for rehabilitation of tracks, quarries and landings, their suitability for different procedures, and their safe use and maintenance

mathematical procedures for estimating and measuring tasks, time and resources, including calculating time to complete tasks

procedures for recording and reporting workplace information

water run-off patterns, soil types and erosion

WHS practices including:

manual handling, including shifting, lifting and carrying

personal protective equipment and clothing

elimination of hazardous materials and substances

plants suitable for revegetation

techniques for managing environmental risks and hazards, including:

uneven and unstable ground

overhead and underground services

excavations

hazardous materials

traffic

insects and animals

fires.

# Assessment Conditions

The following resources must be made available:

tracks, quarries or landings requiring rehabilitation

machinery and equipment necessary for the rehabilitation, and manufacturer’s instructions for their use

dozer

front-end loader

skidder

grader using bucket or blade attachments

personal protective equipment necessary for the rehabilitation

workplace documentation

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47