# Assessment Requirements for FWPCOT3226 Assess timber for manufacturing potential

Release: 1

Assessment Requirements for FWPCOT3226 Assess timber for manufacturing potential

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has visually assessed at least three different types of timber for manufacturing potential for one timber wall frame and one timber roof or floor truss project, and has:

used at least two of the following tools in assessments:

charts

gauges

picture cards

measuring equipment

completed an assessment report for each project, identifying timber characteristics, defects and impacts on nail plate holding ability.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

guidelines, codes and standards for provisions of relevant engineered strength ratings and timber ability to hold nail plates

workplace policies and procedures relating to timber assessment:

labelling of assessed timber products

lines of communicating and reporting

reporting and recording outcomes

orientating components to make best use of timber characteristics

workplace safety and environmental procedures for undertaking timber assessment and  disposal of defective timber products

characteristics, properties and limitations of potential timber products for manufacturing

defects of hardwoods and softwoods timber products and impacts on nail plate holding ability:

wane/want and collapse

bow, spring and twist

cupping and other distortion

knots and knot holes

borer damage

gum/resin veins and pockets

decay

end splits, internal checks and surface checks

low density material

compression failures and fractures

durability

heart in material

joint strength

industry standard timber species and materials and nail plate holding ability:

native timber species

imported timber species

dressed timber

in-the-rough timber

stress and non-stress graded timber

preservative treated timber

laminated veneer coated and treated timber products

beams or laminated beams

industry standard lengths, cross sections and grades, and impact on nail plate holding ability

timber visual assessment techniques

tools used to assess timber products:

charts

guages

picture cards

measuring equipment

industry terminology relating to assessing timber for manufacturing potential.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber fabrication workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

tools to assess timber for manufacturing potential

at least three different types of timber to be assessed

specifications:

acess to work orders and standards and codes of practice related to assessing timber for manufacturing potential

access to workplace safety and environmental policies and procedures applicable to assessing timber in a fabrication workplace

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47