# FWPCOT3228 Plane/sand panels

Release: 1

FWPCOT3228 Plane/sand panels

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT3228B Plane/sand panels, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to plane and sand the the surfaces of panels, optimising board thickness or retaining integrity of veneers. Work is complete in a forest products factory setting and includes operator maintenance.

The unit applies to Timber Products Technician, Production Worker.

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| --- | --- |
| 1. Prepare for planing and sanding | 1.1 Review work order and where required check with appropriate personnel. 1.2 Select appropriate personal protective and other equipment and check for operational effectiveness. 1.3 Obtain type and quantity of panels for planing or sanding from the production process. 1.4 Plan planing or sanding process in line with work order. |
| 2. Plane and sand panels | 2.1 Use equipment in line with organisational safety procedures, manufacturer’s instructions and environmental protection practices. 2.2 Complete pre start-up checks and set planing or sanding thickness and feed rate for optimal product recovery. 2.3 Position material and feed at rate appropriate to machine speed, capacity, board or veneer panel thickness and condition. 2.4 Plane or sand panels to specification and assess panel visually throughout process for thickness, finish quality and veneer faults. 2.5 Coordinate outfeed to ensure efficient recovery of planed or sanded material. 2.6 Check dust extraction equipment regularly and clean to avoid blockages. |
| 3. Redistribute material | 3.1 Grade finished panels for quality and use in line with industry standards. 3.2 Direct material to storage or processing operations in line with organisational procedures. 3.3 Dispose of sub-standard material in line with environmental protection practices. 3.4 Clean work area in line with organisational safety procedures. |
| 4. Complete operator maintenance | 4.1 Follow organisational safety procedures to lock out equipment. 4.2 Check planer blades or sanding belts for bluntness or damage. 4.3 Remove and replace planer blades or sanding belts in line with manufacturer’s recommendations. 4.4 Keep machine area clear of dust and debris in line with organisational safety procedures. 4.5 Record and report production outcomes, equipment faults and maintenance requirements to appropriate personnel. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | identify quantities of required material within work orders and count numbers of panels interpret order sizes and set up equipment to achieve correct dimensions and grades calculate the feed rate to optimise quality and quantity of production output. |
| Oral communication skills to: | ask open and closed probe questions and actively listen to clarify contents of work orders. |
| Reading skills to: | interpret: workplace health and safety and other organisational procedures work orders manufacturers’ instructions and maintenance recommendations. |
| Writing skills to: | complete accurate basic records for production outcomes, equipment faults and maintenance requirements. |
| Planning and organising skills to: | efficiently and logically sequence the stages of preparation and production using work order to guide activities. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPICOT3228B Plane/sand panels

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT3228 Plane/sand panels

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT3228B Plane/sand panels, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to planing and sanding panels

plane and sand panels in readiness for application to panel products or for interim storage

redistribute material on site using designated equipment

conduct operator maintenance on planing and sanding equipment

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

planning or sanding techniques

methods of visual inspection

characteristics of timber and veneer

typical flow of material supply in and out of the planing and sanding area

typical work flow processes and sanding techniques for optimising thickness, finish and quality of timber and veneer panels

typical timber and veneer defects and how they affect ability to produce quality finished product:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

lifting veneer

bonding problems

key features of, and applicable terminology for, industry standard grades for re-classifying quality and use of planed and sanded timber and veneer panels

purpose, features and operation of equipment used to plane and sand panels:

panel planers or thicknessers

surface planers or buzzers

drum sanders

panel sanders

veneer sanders

stroke sanders

dust extraction equipment:

machine specific dust extraction systems with collection bags

hopper-based extraction systems

methods for assessing condition of planer blades and sanding belts

organisational policies and procedures specific to planing and sanding panels:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste

# Assessment Conditions

The following resources must be made available:

planing equipment

sanders including one or more of the following:

drum sanders

panel sanders

veneer sanders

stroke sanders

dust extraction equipment

maintenance tools and equipment for planers and sanders

consumable planer blades and sanding belts

personal protective equipment suitable for planing and sanding panels

commercial panels

manufacturers’ instructions for use and maintenance of equipment

work order with specific instructions for planing and sanding panels

template documents for recording production outcomes, equipment faults and maintenance requirements

organisational policies and procedures for planing and sanding panels.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

#### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47