# Assessment Requirements for FWPCOT3243 Operate a truss press

Release: 1

Assessment Requirements for FWPCOT3243 Operate a truss press

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT3243B Operate a truss press, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

set up components and press timber trusses, in line with an individual work order and construction plan for each job prescribing different dimensions and numbers of trusses

for each of the above jobs, strap, pack and label completed products ready for transportation.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

quality requirements, at a basic level of understanding, of national building codes and standards

environmental protection practices for assembling and pressing timber trusses:

reducing water and energy use

disposing of, recycling and reusing timber and other waste

typical timber defects and how these affect ability to produce a quality finished product:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

typical flow of material supply in and out of the truss pressing area

typical truss assembly sequences and range of techniques used to press trusses

range of industry standard lengths, cross sections and tolerances and applicable terminology

typical  assembly and pressing faults triggering correction to ensure product quality:

incorrectly positioned joints

timber splits from nailing

incorrectly positioned hardware

purpose, features and operation of equipment used to assemble and press timber trusses:

truss presses:

roller press

platen press

C-clamp press

assembly equipment:

measuring equipment

assembly jigs

drills

staple guns

nail guns

compressor or compressed air supply

marking equipment

strapping equipment

appropriate range of cycle times and pressures used to press trusses until structurally ready

purpose, features and use of assembly componentry

organisational procedures specific to operating press trusses:

workplace health and safety with particular emphasis on on equipment lock out and use of personal protective equipment

communication reporting lines

maintaining construction standards

recording and reporting production outcomes and equipment faults

labelling products for identification and transportation.

# Assessment Conditions

The following resources must be made available:

a truss press which includes one or more of the following:

roller press

platen press

C-clamp press

computer-automated press

tools and equipment used to assemble timber trusses:

measuring equipment

assembly jigs

drills

staple guns

nail guns

compressor or compressed air supply

marking equipment

strapping equipment

assembly hardware:

nailing plates

gang nails

nail gun nails

staples

personal protective equipment suitable for operating truss presses

pre-cut sections of timber

consumable strapping, packaging and labelling materials

manufacturers’ instructions for equipment use

work order and construction plan with specifications for assembling and pressing trusses

template documents for recording production outcomes and equipment faults

organisational procedures for operating truss presses.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47