# Assessment Requirements for FWPCOT3244 Cut material to profile

Release: 1

Assessment Requirements for FWPCOT3244 Cut material to profile

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPICOT3244B Cut material to profile, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to cutting material to profile

cut material to profile in line with a work order and prescribed organisational specifications

conduct operator maintenance

stack batches of cut material in line with organisational requirements and WHS regulations

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste

reducing water and energy use

characteristics of timber and timber features

storage systems and labelling procedures including

storage racks

storage bays

bins

stacks

pallet boxes

modularised storage components

temporary stacking bays

storage layout and categories for:

raw materials

stacking finished panels

typical timber defects and how they affect ability to produce quality profiled product:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

range of common profiles and applicable terminology including:

quad

beading

mouldings

shaped beams

architraves

skirting boards

scotia

common cutting patterns used to achieve common profiles

range of feed rates and how they affect production output and finished quality of material

purpose, features and operation of equipment used to cut materials to profile:

measuring equipment

multi-head planing machines (four siders)

spindle moulders

routing machines

shaping machines

machine guides

methods for assessing cutting blade condition

organisational procedures specific to cutting materials to profile:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

The following resources must be made available:

measuring equipment

profiling equipment including one or more of the following:

multi-head planing machines (four siders)

spindle moulders

routing machines

shaping machines

machine guides

maintenance tools and equipment for for removing and replacing cutting blades

consumable profile cutting blades in various shapes and sizes

personal protective equipment suitable for cutting material to profile

suitable material including one or more of the following:

dressed timber

in-the-rough timber

stress and non-stress graded timber

preservative treated timber

medium density fibreboard

laminated veneer

chipboard

fibreboard

coated and/or treated timber products

beams, including laminated beams

manufacturers’ instructions for use and maintenance of equipment

work order with specific instructions for cutting material to profile

template documents for recording production outcomes, equipment faults and maintenance requirements

organisational policies and procedures for cutting material to profile.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47