# Assessment Requirements for FWPCOT3272 Set up and run multi-head moulder/planer to produce simple profiles

Release: 1

Assessment Requirements for FWPCOT3272 Set up and run multi-head moulder/planer to produce simple profiles

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has:

set up and run a multi-head moulder/planer to produce timber with any of the following simple profiles:

dressed all round (DAR) boards, either with a square edge or pencil round edge

skip-dressed boards

in doing the above job:

followed work order and relevant workplace health and safety and environmental protection procedures

selected and fitted cutter heads for a profile specified in a work order

calibrated the moulder/planer to ensure output timber conforms to the required profile

run the moulder/planer

completed workplace reports on set-up, calibration and production activities.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose, features, operating parameters, components and operation of multi-head moulders/planers

equipment used in setting up and running multi-head moulders/planers

types of cutters used to produce simple profiles

types and applications of simple profiles:

DAR timber boards, either with a square edge or pencil round edge

skip-dressed timber boards

cutter head damage and defects:

types

effects

corrective actions

multi-head moulder/planer machine components:

cutters

fences

guides

bed plates

rollers

beam

variables impacting on the finished quality of machined timber:

machine variables, including cutting speed, cutter pitch, feed speed, cutting angles, cutter sharpness

timber variables, including moisture content, grade, age, species

feed rates and how they affect production output and finished quality of timber boards

methods for calculating feed speed

factors that impact on the selection of feed speed:

size

profile

species

typical timber defects and how these impact on the quality of finished product:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

industry standard profiles, lengths, cross-sections and tolerances and applicable terminology

cutting patterns used on boards to achieve maximum volume recovery

methods for assessing cutter condition

types of risk and hazards and mitigation measures associated with dressing boards using multi-head moulders/planers

workplace procedures specific to dressing boards using multi-head moulders/planers:

workplace health and safety, with particular emphasis on equipment lock-out, use of personal protective equipment (PPE) and safe manual handling techniques

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements

environment protection legislation relating to sawmill operations.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing or manufacturing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

multi-head moulder/planer

cutters for simple profiles

tools for setting up and calibrating multi-head moulders/planers

PPE suitable for setting up and running multi-head moulders/planers

specifications:

template documents for recording outcomes of setting up and running multi-head moulders/planers

manufacturer instructions for setting up and running multi-head moulders/planers

work order with specific instructions for setting up and running multi-head moulders/planers

workplace procedures for setting up and running multi-head moulders/planers.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47