# Assessment Requirements for FWPCOT3276 Sharpen cutters using a straight knife grinder

Release: 1

Assessment Requirements for FWPCOT3276 Sharpen cutters using a straight knife grinder

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has sharpened cutters using a straight knife grinder and has:

followed work order and relevant workplace health and safety and environmental protection procedures

cleaned and checked cutters prior to sharpening

prepared the grinder

mounted the cutters and adjusted the grinder

checked and stored the sharpened cutters’

completed workplace reports.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose, features, operating parameters, components and operation of straight knife grinders used to sharpen cutters

equipment used in setting up and running straight knife grinders

types of cutters used to produce simple profiles

types, effects and remedial action for dealing with cutters faults:

warp

cracks

chips

burns

delamination

raised spots

types, effects and remedial action for dealing with grinding wheel faults:

cracks

clogging

effects of incorrect set-up of coolant flow, traversing stops, traversing speed, wheel advance, grinding angle

problems associated with operation of grinder:

burning

incorrect grinding angle

incorrect cutter set-up

defects in cutter edges after sharpening:

burning

chipping

procedures for:

checking and cleaning cutters prior to sharpening

preparing grinder

mounting cutters in grinder

sharpening cutters

storing cutters

types and applications of simple profiles:

dressed all round (DAR) boards, either with a square edge or pencil round edge

skip-dressed boards

types and effects of hazards and control measures associated with using grinders:

grinding wheel disintegration

loose grinding wheel

exposure to grinding coolant

workplace procedures specific to using straight knife grinders:

workplace health and safety, with particular emphasis on equipment lock-out, isolation procedures, machine guarding and use of personal protective equipment (PPE)

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements

environment protection legislation relating to sawmill operations.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing or manufacturing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

straight knife grinder

cutters for simple profiles

tools for setting up straight knife grinder

PPE for sharpening cutters using a straight knife grinder

cutters

lubricant

coolant

specifications:

template documents for recording work outcomes, equipment faults and maintenance requirements

manufacturer instructions for sharpening cutters using a straight knife grinder

work order with specific instructions for sharpening cutters using a straight knife grinder

workplace procedures for sharpening cutters using a straight knife grinder.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47