# Assessment Requirements for FWPCOT3277 Sharpen cutters in head using a profile knife grinder

Release: 1

Assessment Requirements for FWPCOT3277 Sharpen cutters in head using a profile knife grinder

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has sharpened cutters in head using a profile knife grinder and has:

followed work order and relevant workplace health and safety and environmental protection procedures

selected template that met profile requirements

cleaned and checked cutters and cutter head before sharpening

prepared the grinder

mounted and adjusted cutter head and template in the grinder

stored cutter head after sharpening

completed workplace reports.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose, features, operating parameters, components and operation of profile knife grinders used to sharpen cutters in head

equipment used in setting up and running profile knife grinders

purpose of template and specifications

components of cutter head

types of cutters used to produce complex profiles

types, effects and remedial action for dealing with cutter and cutter head faults:

warp

cracks

chips

burns

delamination

raised spots

types of grinder wheels for sharpening various cutter profiles

types, effects and remedial action for dealing with grinding wheel faults:

cracks

clogging

effects of an incorrect set-up of coolant flow, traversing stops, traversing speed, wheel advance, grinding angle

problems associated with the operation of grinder:

burning

incorrect grinding angle

incorrect cutter set-up

defects in cutter edges after sharpening:

burning

chipping

grinding marks

procedures for:

cleaning cutters and cutter head

preparing grinder

mounting and adjusting cutter head and template in the grinder

grinding and honing cutters in head

storing cutter head

cutting angles for specific wood species and desired finish

tolerances for concentricity and weight

types and applications of complex profiles:

tongue and groove flooring

lining board

shiplap

joinery profiles and other detailed mouldings

types and effects of hazards and control measure associated with using grinders:

grinding wheel disintegration

loose grinding wheel

exposure to grinding coolant

workplace procedures specific to using profile knife grinders:

workplace health and safety, with particular emphasis on equipment lock-out, isolation procedures, machine guarding and use of personal protective equipment (PPE)

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements

environment protection legislation relating to sawmill operations.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

profile knife grinder

cutters in head for complex profiles

tools for setting up profile knife grinder

PPE for sharpening cutters in head using a profile knife grinder

coolant

specifications:

template documents for recording work outcomes, equipment faults and maintenance requirements

manufacturer instructions for sharpening cutters in head using a profile knife grinder

work order with specific instructions for sharpening cutters in head using a profile knife grinder

workplace procedures for sharpening cutters in head using a profile knife grinder.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47