# FWPCOT3278 Sharpen cutters in head using a straight knife grinder

Release: 2

FWPCOT3278 Sharpen cutters in head using a straight knife grinder

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Application

This unit of competency describes the skills and knowledge required to sharpen cutters in head using a straight knife grinder.

The unit applies to individuals who sharpen cutters in head on a straight knife grinder for use on a moulder/planer to produce timber with simple profiles such as DAR (dressed all round) boards, either with a square edge or pencil round edge, or skip-dressed boards.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work | 1.1 Review work order to determine job requirements and, where required, seek clarification from appropriate personnel 1.2 Review workplace health and safety procedures, including the use of personal protective equipment, equipment lock-out and safe manual handling techniques 1.3 Identify, assess and take actions to mitigate risks and hazards associated with sharpening cutters in head using a straight knife grinder 1.4 Identify workplace environmental protection procedures for minimising and disposing of waste material, cleaning tools and equipment, and maximising energy efficiency 1.5 Consult with appropriate personnel to ensure that work is coordinated effectively with others in the workplace 1.6 Obtain tools, equipment and testing devices needed for the work, and check for correct operation and safety |
| 2. Clean and check cutter head before sharpening | 2.1 Clean cutter head according to workplace procedures 2.2 Check cutter head for faults and take remedial action according to workplace procedures 2.3 Check, and if required adjust, cutters for position in or on cutter head according to manufacturer specifications |
| 3. Prepare grinder | 3.1 Clean grinder and work area according to workplace procedures 3.2 Select grinding wheel and grinding speed according to workplace procedures 3.3 Check grinding wheel for faults and take remedial action according to workplace procedures 3.4 Fit grinding wheel and check grinder operation according to manufacturer and workplace procedures 3.5 Shape and dress grinding wheel according to manufacturer and workplace procedures 3.6 Set grinding settings according to workplace requirements and manufacturer specifications |
| 4. Mount and adjust cutter head in the grinder | 4.1 Lock cutter head to arbor according to manufacturer specifications 4.2 Set tool rest according to manufacturer specifications 4.3 Set wheel advance according to manufacturer specifications 4.4 Set traversing stops according to manufacturer specifications 4.5 Adjust clearance between grinding wheel and cutters according to manufacturer specifications 4.6 Check and top up coolant level according to manufacturer specifications |
| 5. Sharpen cutters in head | 5.1 Check operation of grinder for problems after the first pass according to workplace procedures 5.2 Maintain coolant flow rate and cleanliness during wet grinding according to workplace procedures 5.3 Ground cutters until the specified edge and clearance angles are obtained across all cutters 5.4 Ground cutter edges to a constant straight line 5.5 Sharpen cutter to specified angle, within permitted tolerances for concentricity and weight, and check that cutter is free from defects 5.6 Hone cutters and remove burrs according to workplace procedures 5.7 Clean grinder after sharpening according to workplace procedures |
| 6. Store cutter head | 6.1 Drain cutter head, if applicable 6.2 Store cutter head according to workplace procedures 6.3 Record and report work outcomes, equipment faults and maintenance requirements to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Numeracy | Measures finished dimensions against specifications and allowable tolerances |
| Navigate the world of work | Understands main tasks, responsibilities and boundaries of own role |
| Interact with others | Uses modes of communication suitable to purpose to confirm and clarify understanding |
| Get the work done | Recognises and responds to routine problems |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3278 Sharpen cutters in head using a straight knife grinder Release 2 | FWPCOT3278 Sharpen cutters in head using a straight knife grinder Release 1 | Minor edit to Application | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47