# FWPCOT3292 Calibrate and maintain scanning equipment used in sawmilling operations

Release: 1

FWPCOT3292 Calibrate and maintain scanning equipment used in sawmilling operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Application

This unit of competency describes the skills and knowledge required to calibrate, adjust and test different forms of scanning equipment used in sawmilling operations, including optical (1–3 dimension), X-ray, microwave, multispectral and acoustic scanning equipment. It includes working safely, identifying the capability of scanning equipment in detecting timber characteristics and defects, following calibration procedures, and reporting on calibration outcomes.

The unit applies to individuals who calibrate different forms of scanning equipment used in sawmilling operations.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare to calibrate scanning equipment | 1.1 Review work order to determine job requirements and, where required, seek clarification from appropriate personnel 1.2 Review workplace health and safety procedures, including the use of personal protective equipment, equipment lock-out and safe manual handling techniques 1.3 Identify, assess and take actions to mitigate risks and hazards associated with working with scanning equipment 1.4 Identify and implement workplace procedures for minimising waste material and maximising energy efficiency 1.5 Obtain tools, equipment and testing devices needed for the work, and check for correct operation and safety |
| 2. Calibrate scanning equipment | 2.1 Identify capability and resolution of scanning equipment in detecting timber characteristics and defects 2.2 Identify calibration procedures and calibration points for scanning equipment 2.3 Identify and take into account factors that may impact on the accuracy of scanning equipment during the calibration process 2.4 Test and adjust scanning equipment as necessary to ensure equipment meets calibration requirements 2.5 Carry out calibration of scanning equipment efficiently without waste of materials or damage to apparatus, the surrounding environment or services, and using sustainable energy principles |
| 3. Verify calibration outcome | 3.1 Conduct a test for a benchmark object (the standard) to confirm accuracy and desired outcome from the process 3.2 Repeat calibration procedure if accuracy is not achieved 3.3 Record all calibration data accurately according to workplace and manufacturer requirements |
| 4. Complete calibration activities | 4.1 Document calibration status and report out-of-calibration scanning equipment 4.2 Lock out calibration scanning equipment according to workplace and safety requirements 4.3 Perform routine maintenance of scanning equipment according to workplace and manufacturer instructions 4.4 Clean work area according to workplace and safety requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Extracts key information from operational manuals to determine software parameters of scanning equipment |
| Writing | Prepares routine written reports using required format, language and structure |
| Oral communication | Employs active listening and questioning to clarify content of work orders |
| Numeracy | Uses standard measures to record calibration outcomes Interprets graphical and statistical data used in calibration process |
| Navigate the world of work | Understands main tasks, responsibilities and boundaries of own role |
| Interact with others | Uses modes of communication suitable to purpose to confirm and clarify understanding |
| Get the work done | Recognises and responds to routine problems |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3292 Calibrate and maintain scanning equipment used in sawmilling operations | Not applicable | New unit | No equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47