# Assessment Requirements for FWPCOT3295 Assess and maintain cutter performance

Release: 1

Assessment Requirements for FWPCOT3295 Assess and maintain cutter performance

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has successfully assessed and diagnosed three different problems related to cutter and cutter blocks performance for moulders or planers.

In doing the above, the individual has:

followed work order and relevant workplace health and safety and environmental protection procedures

assessed performance of cutter and assembly components

inspected timber and cutter and assembly components

identified safe course of action for rectifying problem

reported on problem, fault diagnosis process and rectification outcomes.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose, features and operation of cutter, cutter block and assembly components on moulders and planers

typical causes of damage and poor cutter performance:

forcing timber through the machine

damage from foreign matter

incorrect operating speeds

oversized material removal

incorrect setting of machine

poor sharpening or setting of cutters or knives

worn or damaged components

methods for:

assessing condition of cutters and assembly components

diagnosing causes of damage and poor performance

rectifying problems and improving cutter performance

types of risk and hazards and mitigation measures associated with assessing and maintaining cutter performance

workplace procedures specific to assessing and maintaining cutter performance:

workplace health and safety, with particular emphasis on equipment lock-out and use of personal protective equipment (PPE)

communication reporting lines

recording and reporting cutting problems

diagnosis process and rectification outcomes

disposing of material.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing or manufacturing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

one or more of the following machines and associated cutters: moulder, planer

hand tools to carry out the work

PPE suitable for assessing and maintaining performance of cutters

specifications:

records of equipment fault and cutting performance

template documents for recording and reporting cutting problems, the fault diagnosis process and rectification outcomes

workplace procedures for assessing and maintaining performance of cutters and associated equipment

manufacturer instructions for use, repair and maintenance of cutters and associated equipment.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47