# Assessment Requirements for FWPCOT3308 Assemble timber wall frames

Release: 1

Assessment Requirements for FWPCOT3308 Assemble timber wall frames

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has assembled a minimum of 50 lineal metres of timber wall frames that include:

load-bearing and non-load-bearing frames

internal, external corners and T-intersections

strapping, packing, labelling and pre-slinging completed products ready for despatch.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

workplace quality requirements relevant to assembling timber wall frames

workplace environmental policy and procedures for disposing of, recycling and reusing waste materials

workplace safety requirements relevant to assembling timber wall frames:

safe operation of machinery, tools and equipment

exposure to excessive noise

hazardous manual tasks

hazard identification and controls

use of personal protective equipment (PPE)

exposure to dust

processes, procedures and techniques for:

flow of material supply in and out of the wall frame assembly area

assembly sequences using a jig or assembly table

stacking and storing materials

labelling products for identification and transportation

stacking, binding and protecting finished products ready for despatch

workplace communication and reporting lines

recording and reporting assembly outcomes and equipment faults

functional and operational features of tools, equipment and machinery:

measuring and marking equipment

electric circular saws

docking saws

drills

assembly jigs

assembly table

staple and nail guns

compressor or compressed air supply

strapping equipment

characteristics, properties and limitations of timber products and components:

hardwood, softwood and manufactured timber products

metal connector plates and straps

timber defects:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

industry terminology related to timber wall assembly:

studs

top and bottom plates

lintels

trimmers

nogging

bracing

typical assembly faults triggering correction to ensure product quality:

incorrectly positioned joints

timber splits from nailing

incorrectly positioned hardware.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber frame fabrication facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

timber products and components to assemble timber frames

equipment, tools and machinery to assemble wall frames and finalise frames for despatch

specifications:

access to work order or fabrication plan detailing frame assembly to be undertaken

access to workplace safety and environmental policies and procedures applicable to assembling timber wall frame

relationships:

supervisor or team member.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47