# FWPCOT3308 Assemble timber wall frames

Release: 1

FWPCOT3308 Assemble timber wall frames

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to assemble construction-ready timber wall frame sections. It includes packing and strapping frames in readiness for transport to a construction site.

The unit applies to frame fabricators, timber manufactured products technicians, jig setters, machine and saw operators who take responsibility for organising and completing tasks assigned to them without close supervision.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for assembly | 1.1 Determine job requirements from work order or fabrication plan and, where required, confirm with appropriate person 1.2 Confirm safety requirements for timber frame assembly according to workplace safety procedures 1.3 Identify type and determine quantity of material required for assembly of frame according to fabrication plan 1.4 Determine type of connectors and bracing for frames according to fabrication plan 1.5 Select, fit and use appropriate personal protective equipment (PPE) 1.6 Select appropriate tools and equipment, check for serviceability, and report any faults or damage 1.7 Determine job priorities and sequence job tasks according to fabrication plan |
| 2. Set up for timber frame assembly | 2.1 Conduct pre-start check on equipment and machinery according to manufacturer instructions and workplace procedures 2.2 Prepare and set up assembly table or jigs required for assembling frames 2.3 Select pre-cut timber components and accurately position pre-cut members to align with marking or jigs 2.4 Reject and dispose of defective material according to workplace safety and environmental procedures 2.5 Use tools to firmly fix timber components within acceptable tolerances 2.6 Check frame for parallel and square, and fix specified bracing according to fabrication plan 2.7 Locate connector plates and strapping from job specification, and fix securely |
| 3. Prepare products for despatch | 3.1 Check finished product for quality and label according to work order and workplace procedures 3.2 Remove frame from jigs and safely stack in designated location 3.3 Strap, pack, label, protect from damage and pre-sling finished products ready for transportation 3.4 Minimise wastage of strapping and packaging materials and maximise use of recycled materials according to workplace environmental procedures 3.5 Record and report production outcomes and problems according to workplace reporting procedures |
| 4. Clean up | 4.1 Clean work area and recycle, reuse or dispose of waste according to workplace environmental procedures 4.2 Clean and maintain tools, equipment and machinery, and report any faults or damage according to workplace procedures 4.3 Return and safely store tools and equipment in designated areas |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Oral communication | Report problems, faults or damage clearly and concisely using appropriate terminology |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3308 Assemble timber wall frames | FWPCOT3241 Assemble timber wall frames | Application clarified Elements renamed Performance Criteria re-sequenced Foundation Skills and Assessment Requirements updated | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47