# FWPCOT3325 Operate four wheel drive vehicle on unsealed roads

Release: 1

FWPCOT3325 Operate four wheel drive vehicle on unsealed roads

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Application

This unit of competency describes the outcomes required to safely operate a four wheel drive vehicle on unsealed roads.

The unit applies to individuals who operate four wheel drive vehicles in a variety of work settings and industry sectors, including forestry, agriculture, construction, tourism and energy.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

Individuals must hold a driver licence or a provisional driver licence prior to commencing this unit of competency.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for operations | 1.1 Access, interpret and apply vehicle manufacturer information relevant to operating a four wheel drive vehicle on unsealed roads 1.2 Identify the structural and handling differences between a conventional two wheel drive vehicle and a four wheel drive vehicle 1.3 Determine type of operation and ensure safe operating conditions 1.4 Identify, fit and use suitable personal protective equipment 1.5 Perform pre-start checks 1.6 Establish and check communication with others, including convoy procedures 1.7 Secure loads according to safety, environmental and regulatory requirements 1.8 Research weather and topography for a planned trip on unsealed roads |
| 2. Operate vehicle on unsealed roads | 2.1 Check and change tyre pressure suitable for unsealed road according to workplace procedures 2.2 Continually monitor the track to identify hazards, assess risks and select the best route according to safety and environmental requirements 2.3 Identify situations where the engagement of four wheel drive is required and engage front hubs or centre differential lock where fitted 2.4 Use range, gear, speed, driving and braking technique appropriate to surface conditions while maintaining control of vehicle at all times 2.5 Check brake operation and undercarriage after operating vehicle on unsealed road conditions 2.6 Park and shut down vehicle |
| 3. Ascend and descend slopes less than 15 degrees | 3.1 Inspect intended vehicle path before negotiation of slope 3.2 Use appropriate range, gear, speed, driving and braking technique to negotiate a slope while maintaining control of vehicle 3.3 Use appropriate braking methods, user applied and vehicle controlled function, where fitted, to descend a slope 3.4 Maintain traction and minimise wheel spin 3.5 Apply appropriate automatic and manual techniques when vehicle cannot proceed any further on a slope |
| 4. Complete operation | 4.1 Conduct a post-trip inspection of vehicle 4.2 Complete post-trip documentation, including maintenance and minor repairs reports |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Read and comprehend basic factual information on four wheel drive vehicle equipment labels and in manufacturer manuals |
| Writing | Complete workplace forms using required format, industry terminology and structure |
| Oral communication | Use effective verbal communication techniques to convey instructions to people involved in four wheel drive operation |
| Numeracy | Estimate time of a journey and fuel consumption |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3325 Operate four wheel drive vehicle on unsealed roads | FWPCOT3259 Operate a four wheel drive on unsealed roads | Updated unit title Updated Elements and Performance Criteria Updated and added new Knowledge Evidence and Performance Evidence Reworded Foundation Skills Updated Assessment Conditions | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT3325 Operate four wheel drive vehicle on unsealed roads

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has performed each of the following tasks while driving a four wheel drive vehicle on unsealed roads:

driven three separate ascents and three separate descents less than 15 degrees. In conducting this task, the individual has:

selected appropriate range and gearing

used foot brake effectively to control descent speed

used vehicle hill descent or similar function where available

used the most appropriate wheel placement

used on one occasion traction aids and brakes when driving ascents and descents less than 15 degrees

performed on two occasions a forward and a reverse stop start recovery on an incline according to the type of transmission

demonstrated on one occasion the setting of vehicle controls for operation on unsealed roads. This may include:

the use of free wheel hubs if fitted

selection of low range in either manual or automatic vehicles

use of terrain selection system if fitted

use of differential locks if fitted

activation of traction control or terrain selection mode if fitted and where effective

driven vehicle and operated vehicle controls and technology to avoid traction loss on unsealed roads in three of the following conditions:

loose gravel

ruts

loose material in centre and sides of track

corrugations

mud

rocky

sand

snow

side slopes

rutted

water crossing

fitted and used personal protective equipment (PPE).

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

structural differences between conventional two wheel drive and a four wheel drive, including:

purpose and use of front wheel hubs

factors affecting tyre selection and pressure

environmental protection requirements affecting four wheel drive operations

common unsealed road surface hazards:

loose gravel

ruts

loose material in centre and sides of track

corrugations

pre-start checks for four wheel drive vehicle

types of communication equipment, including:

personal location beacon (PLB)

phone

radio

vehicle/personnel tracking devices

safe operating conditions for a four wheel drive vehicle:

interpreting weather conditions to determine suitability for driving

selecting routes to minimise damage to the environment

visibility

loading of four wheel drive, including techniques to reduce overloading

braking techniques

cornering techniques

types of environmental risks and hazards associated with operating a four wheel drive vehicle on unsealed roads, including:

rocks and other unexpected obstacles

damage to vehicle

animals and objects in vehicle path

slippery surfaces

overhead tree limbs

other vehicles

fallen trees, limbs and logs

flood

poor visibility

tyre pressure management principles for varying terrain types

principles for effective self-recovery of vehicle via driving skills (not the use of recovery equipment such as straps or winches)

communication channels and protocols

techniques for travelling in convoy

procedures for recording and reporting workplace information.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a work environment or an environment that accurately represents workplace conditions

resources, equipment and materials:

site with access to unsealed roads

manual or automatic four wheel drive vehicle or all-wheel drive vehicle

PPE required for operating a four wheel drive on unsealed roads

relevant personnel for the purposes of communicating information

specifications:

workplace safety and environmental policies and procedures applicable to forestry operations.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47