# FWPCOT3326 Recover four wheel drive vehicle

Release: 2

FWPCOT3326 Recover four wheel drive vehicle

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with Forest and Wood Products Training Package Version 8.0. |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Application

This unit of competency describes the skills and knowledge required to use recovery equipment to recover four wheel drive vehicles in difficult terrain conditions.

The unit applies to individuals who recover four wheel drive vehicles in a variety of work settings and industry sectors, including forestry, agriculture, construction, tourism and energy.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

Individuals must hold a driver licence or a provisional driver licence prior to commencing this unit of competency.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Identify loads, ratings and equipment serviceability related to four wheel drive vehicle recovery | 1.1 Determine the Gross Vehicle Mass (GVM) and estimate the recovery load on the vehicle using a recognised formula 1.2 Identify the rating and labelling of recovery gear 1.3 Identify rated recovery points on a vehicle 1.4 Determine the appropriate equipment to be used given the GVM, estimated recovery load and the equipment rating 1.5 Determine serviceability of recovery equipment required for task |
| 2. Plan recovery | 2.1 Identify and follow work health and safety procedures, including selection and use of personal protective equipment relevant to recovery operations 2.2 Identify, assess and take actions to mitigate risks and hazards associated with recovery operations 2.3 Identify and implement workplace environmental protection procedures associated with recovery operations 2.4 Identify emergency procedures to follow in the event of accident or incident 2.5 Locate and survey site to determine recovery strategy and establishment of an exclusion zone 2.6 Inform appropriate personnel of location of vehicle to be recovered and recovery strategy |
| 3. Perform kinetic rope or strap recovery | 3.1 Establish and maintain communications with appropriate personnel during recovery operation 3.2 Establish when and by what means to start and stop the recovery 3.3 Lay out the kinetic rope/strap prior to recovery 3.4 Fit the kinetic rope/strap to an approved recovery point/device that is suitably rated for use with the strap 3.5 Demonstrate the joining of two kinetic ropes and/or straps 3.6 Identify where to place dampeners 3.7 Perform a safe recovery using the kinetic rope/strap |
| 4. Perform winch recovery | 4.1 Establish and maintain communications with appropriate personnel during recovery operation 4.2 Establish when and how to start and stop the recovery 4.3 Lay out all the equipment to be used, including the dampeners 4.4 Perform winch recovery using a single line pull 4.5 Perform winch recovery using a double line pull using a snatch block or pulley ring 4.6 Perform winch recovery using a change of direction pull using a snatch block or pulley ring |
| 5. Use other recovery tools and equipment | 5.1 Use a spade for vehicle recovery 5.2 Use traction boards for vehicle recovery |
| 6. Maintain vehicle recovery equipment | 6.1 Clean, carry out operator maintenance, check serviceability and store recovery equipment after use according to manufacturer requirements 6.2 Record equipment maintenance according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret basic factual information on tool and equipment labels and in manufacturer manuals |
| Writing | Complete workplace forms using required format, industry terminology and structure |
| Oral communication | Use effective verbal communication techniques to convey instructions to people in vicinity location of recovery operation |
| Numeracy | Apply numeric skills to check rating of recovery equipment |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3326 Recover four wheel drive vehicle Release 2 | FWPCOT3326 Recover four wheel drive vehicle Release 1 | Minor edits to Performance Evidence for providing clarification | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT3326 Recover four wheel drive vehicle

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with Forest and Wood Products Training Package Version 8.0. |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

performed two kinetic rope and/or strap recoveries

performed one of each of the following winch recoveries:

single line

double line

change of direction.

In conducting each of the above tasks, the individual has:

estimated the recovery load using a recognised formula

determined the Gross Vehicle Mass (GVM) of the vehicle/s in the recovery

determined the rating of the equipment being used

determined the most appropriate recovery strategy to be used

assessed why the vehicle is bogged, taken action to ensure vehicle is in four wheel drive, taken action to clear obstructions from the vehicle, taken action to use traction boards where available, determined safest method of recovery if straps or winches are to be used

determined the appropriate equipment to be used given the GVM, equipment rating and chosen strategy.

There must also be evidence that on one occasion the individual has joined two kinetic ropes and/or straps.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

safety procedures during recovery operations

recognised formula used to estimate the recovery load and what variables apply to this formula

reasons and procedures for establishing exclusion zone

Gross Vehicle Mass (GVM):

identifying which vehicle’s GVM should be used in relation to kinetic recovery

ratings used on recovery equipment:

MBS (Minimum Breaking Strength) of ropes, straps and soft shackles

WLL (Working Load Limit) of hard shackles

drag chains

pulley blocks

pulley rings

winches (wire and/or rope)

anchor point selection

number, location and weighting of strap dampeners

rated recovery points:

characteristics and purpose of rated recovery points

location of rated recovery points

what is and what is not a rated recovery point

kinetic recovery:

kinetic recovery procedure

manufacturer recommendations on recovery techniques and equipment

winches:

types, main components, ratings and manufacturer recommendation for use

effect of the layers of wrap of rope or wire on a winch drum

effects of a ring or snatch block on the load on a winch

impact of angle of rope or cable on recovery operation

procedures for easing the load on a winch

the reasons for not driving while winching

maintenance of recovery gear:

straps/ropes

winches.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a work environment or an environment that accurately represents workplace conditions

work environment must include varied terrains and conditions from which to recover vehicle

resources, equipment and materials:

four wheel drive to recover

rated recovery equipment for four wheel drive vehicle

relevant personnel for the purposes of communicating information

personal protective and safety equipment required for recovering a four wheel drive vehicle

specifications:

workplace safety and environmental policies and procedures applicable to the recovery of four wheel drive vehicle.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47