# Assessment Requirements for FWPCOT3339 Grade wood product visually

Release: 1

Assessment Requirements for FWPCOT3339 Grade wood product visually

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has graded, sorted and marked one product selected from the following list for one production run according to workplace procedures:

chipboard

coated and/or treated timber products

cross arms for overhead lines

crossings

debarked logs

decking for wharves and bridges

dressed timber

fibreboard

in-the-rough timber

laminated veneer

medium density fibreboard

mine guides

piles

plywood

poles

preservative treated timber

recycled timber

sleepers

stress and non-stress graded timber

timber from imported species

timber from native species

other engineered wood products.

In performing each of these tasks, the individual has:

assessed wood product characteristics visually for strength and/or appearance

complied with legislation, standards and codes of practice relevant to visual grading operations

measured wood product moisture content and length requirements and tolerances against allowable limits for the wood product type and end-use application

identified defects in wood products

identified products that require docking, regrading or reworking and those that are rejected

branded, labelled, stamped or marked each graded wood product to indicate compliance with relevant Australian Standard and/or CodeMark certification requirements

safely used workplace equipment for lifting and moving materials

arranged reuse, recycling or disposal of sub-standard wood product

coordinated movement of graded material to storage or processing operations

reported grading outcomes to appropriate personnel.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

markings used on wood products to identify grades

key requirements of legislation, standards and codes of practice relevant to visual grading operations, including:

Australian Standards

CodeMark certification

industry standards for cross-sections and lengths for different types of wood products

grade classifications for different types of wood products

typical flow of material supply in and out of the grading area

methods to label wood products for storage locations, and common terms used

purpose, features and operation of equipment used to grade wood products, including moisture meters

workplace health and safety procedures, with particular emphasis on use of personal protective equipment (PPE) and safe manual handling techniques

types of risk and hazards and mitigation measures associated with grading and sorting activities

equipment used for lifting and moving wood products

types of timber products for grading and sorting

methods of visual estimation, grading, colour identification and tagging of different types of wood products

procedures for sorting, marking, distribution and storage of different types of wood products

methods of assessing wood products for number and frequency of defects

methods for determining and recording moisture content of wood products

wood products defects, including:

warp

wane

cupping

shakes

insect defects

loose or missing knots

resin pockets

dents

loose surface

lifting coatings

thin or missing coatings

chipped edges

splits

voids

splintered or missing edges

pin holes

criteria for disposal, recycling and reuse of sub-standard wood products

methods for safe disposal of waste preservative treated timber.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing plant, a timber product manufacturing plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

moving equipment

PPE suitable for grading, sorting and marking wood products

labelling materials

three different wood products for grading

storage locations for graded wood products

specifications:

workplace procedures for grading and sorting activities.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47