# FWPCOT3339 Grade wood product visually

Release: 1

FWPCOT3339 Grade wood product visually

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |  |

# Application

This unit of competency describes the skills and knowledge required to visually assess the characteristics and defects of wood products, including recycled timber, and establish whether the strength and/or appearance properties of the product are consistent and within the grade specifications required for end-use application.

The unit applies to individuals who visually grade wood products, including recycled timber, in a wood products production facility or other work environments.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common Technical (COT)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for grading wood product | 1.1 Determine job requirements from work order or specifications and, where required, seek clarification from appropriate personnel 1.2 Confirm workplace health and safety and environmental protection practices and/or procedures for task, including use of personal protective equipment, lifting and moving equipment and safe manual handling techniques 1.3 Confirm legislation, standards and codes of practice relevant to visual grading operations 1.4 Identify and assess hazards and take actions to mitigate risks related to grading operations 1.5 Confirm type and quantity of wood product to be graded 1.6 Identify procedures for storage and subsequent processing of wood product 1.7 Select equipment to assist in grading wood product visually, and check for operational effectiveness |
| 2. Grade wood product visually | 2.1 Assess wood product characteristics visually for strength and/or appearance by identifying and measuring defects against performance requirements for wood product type and end-use application according to workplace grade specifications and relevant Australian Standards or industry codes of practice 2.2 Measure wood product moisture content and length requirements and tolerances against allowable limits for wood product type and end-use application according to workplace grade specifications relevant Australian Standards or industry codes of practice 2.3 Determine and mark defects and wood products that require docking, regrading or reworking and those that are rejected 2.4 Brand, label, stamp or mark each graded wood product to indicate compliance with relevant Australian Standard and/or CodeMark certification requirements 2.5 Record wood product grading outcomes |
| 3. Distribute graded wood product | 3.1 Direct material to storage or processing operations according to workplace procedures 3.2 Arrange for reuse, recycling or disposal of sub-standard wood product according to workplace environmental protection practices and procedures 3.3 Monitor on-site movement of wood product to achieve intended flow 3.4 Record distribution problems and report to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret routine workplace documents and industry grading standards |
| Writing | Produce basic clear labels for timber identification Complete accurate basic reports on grading outcomes |
| Oral communication | Ask questions and actively listen to clarify work orders and job tasks and report workplace issues |
| Numeracy | Complete routine grading records involving numerical grades, dimensions and quantities |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPCOT3339 Grade wood product visually | FWPCOT3240 Grade heavy structural/engineered products | Redesigned unit that includes content from FWPCOT3240 Grade heavy structural/engineered products, FWPCOT3284 Grade, sort and mark timber materials and FWPSAW2201 Grade round poles and debarked logs | Not equivalent |
| FWPCOT3339 Grade wood product visually | FWPCOT3284 Grade, sort and mark timber materials | Redesigned unit that includes content from FWPCOT3240 Grade heavy structural/engineered products, FWPCOT3284 Grade, sort and mark timber materials and FWPSAW2201 Grade round poles and debarked logs | Not equivalent |
| FWPCOT3339 Grade wood product visually | FWPSAW2201 Grade round poles and debarked logs | Redesigned unit that includes content from FWPCOT3240 Grade heavy structural/engineered products, FWPCOT3284 Grade, sort and mark timber materials and FWPSAW2201 Grade round poles and debarked logs | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPCOT3339 Grade wood product visually

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has graded, sorted and marked one product selected from the following list for one production run according to workplace procedures:

chipboard

coated and/or treated timber products

cross arms for overhead lines

crossings

debarked logs

decking for wharves and bridges

dressed timber

fibreboard

in-the-rough timber

laminated veneer

medium density fibreboard

mine guides

piles

plywood

poles

preservative treated timber

recycled timber

sleepers

stress and non-stress graded timber

timber from imported species

timber from native species

other engineered wood products.

In performing each of these tasks, the individual has:

assessed wood product characteristics visually for strength and/or appearance

complied with legislation, standards and codes of practice relevant to visual grading operations

measured wood product moisture content and length requirements and tolerances against allowable limits for the wood product type and end-use application

identified defects in wood products

identified products that require docking, regrading or reworking and those that are rejected

branded, labelled, stamped or marked each graded wood product to indicate compliance with relevant Australian Standard and/or CodeMark certification requirements

safely used workplace equipment for lifting and moving materials

arranged reuse, recycling or disposal of sub-standard wood product

coordinated movement of graded material to storage or processing operations

reported grading outcomes to appropriate personnel.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

markings used on wood products to identify grades

key requirements of legislation, standards and codes of practice relevant to visual grading operations, including:

Australian Standards

CodeMark certification

industry standards for cross-sections and lengths for different types of wood products

grade classifications for different types of wood products

typical flow of material supply in and out of the grading area

methods to label wood products for storage locations, and common terms used

purpose, features and operation of equipment used to grade wood products, including moisture meters

workplace health and safety procedures, with particular emphasis on use of personal protective equipment (PPE) and safe manual handling techniques

types of risk and hazards and mitigation measures associated with grading and sorting activities

equipment used for lifting and moving wood products

types of timber products for grading and sorting

methods of visual estimation, grading, colour identification and tagging of different types of wood products

procedures for sorting, marking, distribution and storage of different types of wood products

methods of assessing wood products for number and frequency of defects

methods for determining and recording moisture content of wood products

wood products defects, including:

warp

wane

cupping

shakes

insect defects

loose or missing knots

resin pockets

dents

loose surface

lifting coatings

thin or missing coatings

chipped edges

splits

voids

splintered or missing edges

pin holes

criteria for disposal, recycling and reuse of sub-standard wood products

methods for safe disposal of waste preservative treated timber.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing plant, a timber product manufacturing plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

moving equipment

PPE suitable for grading, sorting and marking wood products

labelling materials

three different wood products for grading

storage locations for graded wood products

specifications:

workplace procedures for grading and sorting activities.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47