# FWPFIR4001 Conduct tree hazard assessment post-fire

Release: 1

FWPFIR4001 Conduct tree hazard assessment post-fire

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Application

This unit of competency describes the skills and knowledge required to conduct hazard assessment of fire affected or fire damaged trees during the early stages of bushfire recovery and make recommendations on risk treatment.

The unit applies to individuals authorised by emergency management agencies to assess tree hazard during the early stages of bushfire recovery operations.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation, standards and industry codes of practice that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Fire Control (FIR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work in bushfire recovery operations | 1.1 Maintain up-to-date information on topography, hazards and weather conditions in bushfire recovery area 1.2 Maintain situational awareness in bushfire recovery area 1.3 Follow health and safety environmental protection practices and/or procedures, including use of personal protective equipment and hazard and risk control measures, relevant to bushfire recovery operations 1.4 Confirm chain of command and communication protocols that apply in bushfire recovery operations |
| 2. Identify, mark and isolate tree hazard | 2.1 Conduct hazard assessment of route to and areas around tree to determine it is safe to approach 2.2 Determine location and position of tree in relation to site topography and other hazards at site 2.3 Establish risk to self and others posed by tree hazard and other factors in immediate vicinity of the tree 2.4 Mark tree, if safe to do so, using spray paint according to industry guidelines and agency approved symbols 2.5 Isolate tree with barrier tape according to industry guidelines and agency approved procedures |
| 3. Assess tree hazard | 3.1 Assess visually extent of fire damage to crown, trunk and root from the appropriate safe distance 3.2 Identify structural and physiological defects in tree 3.3 Assess external factors that may cause tree or its limbs and branches to fall 3.4 Determine risks posed by tree according to workplace procedures 3.5 Identify tree hazard class according to industry guidelines and agency approved procedures 3.6 Identify potential environmental, cultural and economic values of tree and, where required, consult with appropriately qualified person on risk treatment options 3.7 Determine appropriate risk treatment to mitigate tree hazard according to industry guidelines and agency approved procedures 3.8 Record and report outcomes of tree assessment to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret routine text and pictorial workplace documentation relevant to identification and treatment of fire damaged or fire affected hazardous trees |
| Oral communication | Use appropriate vocabulary including technical language directly relevant to role |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPFIR4001 Conduct tree hazard assessment post-fire | Not applicable | The unit has been created to address a skill or task required by industry that is not covered by an existing unit | Newly created |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPFIR4001 Conduct tree hazard assessment post-fire

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has conducted a hazard assessment of five fire damaged or fire affected trees, which include two different species.

In performing each of these tasks, the individual has:

complied with health and safety requirements, including use of personal protective equipment

conducted a dynamic risk assessment of route to and areas around tree

identified risks to self and others posed by tree according to workplace procedures

identified ecological, cultural and economic value of tree

assessed and determined treatment options for tree

reported on assessment and treatment options for tree.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

health and safety work procedures relevant to bushfire recovery operations

workplace hazards and associated risks and controls in bushfire recovery operations, including those associated with fire affected and fire damaged trees

tree species and effects of fire on different trees species

hazards associated with fire affected and fire damaged trees

factors influencing assessment of tree hazard risk and the values in retaining the tree, including:

structural characteristics of trees

likely timeframe in which the tree, limb or branch might be expected to fall

tree exposure to causal factors and external influences of tree hazard

quantity of trees to be assessed

potential of tree to interact with other hazards or assets (e.g., power lines, buildings)

operational context

possible risk control measures that may be applied in context

cultural, social or economic values associated with a tree or its immediate surroundings

ecological values at the site

external factors that increase potential tree hazard, including:

wind exposure

drought

stress and insect infestation

excessive drainage problems from land management operations

exposure to vibration from heavy plant or machinery

other hazards at the site that increase potential tree hazard, including:

gas supplies

water supplies

powerlines (above and below ground)

adjacent buildings

features of adjacent trees

terrain features

characteristics of hazardous trees, including:

dead and/or decaying tree and/or major branches

evidence of previous damage, including burn or mechanical processes

suspected loose or broken branches

evidence of longitudinal or torsion fractures

evidence of roots lifting or disturbed root system

significant lean with indicators of failure

cavities and hollows in the main trunk and root buttress area

industry guidelines and agency approved policies and procedures for tree hazard management in bushfire recovery regarding:

tree hazard isolation

tree hazard marking and mapping symbols

tree hazard classes – clear and present danger tree (CPD), potential clear and present danger trees (potential CPD)

tree hazard treatment – removal of tree or removal of branch/limb, exclusion, communication of ongoing risk to relevant qualified personnel

characteristics of culturally significant trees, including Aboriginal sacred trees, and workplace procedures for managing significant trees, including who to contact if a culturally significant tree is identified

techniques for recording position of trees using Global Navigation Satellite System (GNSS)

reporting procedures associated with tree hazards.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a work environment or an environment that accurately represents workplace conditions

resources, equipment and materials:

fire damaged or fire affected trees

communications systems relevant to work activity

equipment and materials relevant to work activity

personal protective equipment relevant to work activity

GNSS

specifications:

workplace health and safety and environmental protection policies and/or procedures relevant to work activity

workplace documentation for recording information relevant to work activity.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47