# Assessment Requirements for FWPHAR2209 Trim and cut harvested trees

Release: 1

Assessment Requirements for FWPHAR2209 Trim and cut harvested trees

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has assessed, planned and conducted the trimming and cutting of six felled trees using a chainsaw to optimise time, volume recovery and quality.

In performing this work, there must be evidence that the individual:

demonstrated three or more cut types from the list below:

bridging cuts

swinging cuts

side bind cuts

boring cuts

wedge cuts

ripping cuts

step cuts

followed workplace policy and procedures, current workplace health and safety legislation, regulations and related industry standards and codes of practice applicable to cutting felled tree operations

checked and sharpened or changed a saw chain on one occasion according to manufacturer instructions.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

chainsaw cutting techniques used to trim and cut felled trees:

bridging cut

swinging cut

side bind cut

boring cut

wedge cut

ripping cut

step cut

felled tree stresses and how they affect tree cutting activities:

tension (fibres being stretched)

compression (fibres being squashed)

neutral fibres

optimisation in trimming and cutting operations:

time

value recovery

manageable removal of logs

cutting patterns used to optimise time and recovery of product from felled trees

cutting sequences used to maintain control of cut sections and minimise cutting problems

hazards and risks associated with site conditions and effects on chainsaw operations

methods to identify and explain how timber defects may affect the chainsaw operation

industry standards:

lengths and tolerances and applicable terminology

grades for log and applicable terminology

methods for assessing chain condition

workplace policies and procedures specific to tree trimming and cutting activities:

workplace health and safety, with particular emphasis on equipment lock-out, use of personal protective equipment (PPE) and safe manual handling

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements

codes of practice

harvest plans

environmental protection practices for harvesting operations:

soil and water protection

cleaning plant, tools and equipment

disposing of, recycling and reusing timber and other waste.

# Assessment Conditions

Assessment of the skills for this unit of competency must take place under the following conditions:

physical conditions:

competency is to be assessed in a commercial forest operation or a simulated environment that accurately reflects performance in a real workplace setting

resources, equipment and materials:

chainsaw and accessories

suitable felled trees to trim and cut

PPE required in trimming and cutting felled trees

communication system

specifications:

access to work order or instruction detailing cutting and trimming operations to be carried out by chainsaw operator

access to workplace safety and environmental protecting policies and procedures applicable to trimming and cutting operations and chainsaw use

access to workplace standard operating procedures or operator manual for chainsaws.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47