# FWPHAR3201 Monitor log recovery (rigging slinger)

Release: 2

FWPHAR3201 Monitor log recovery (rigging slinger)

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Application

This unit of competency describes the outcomes required to plan for and navigate the extraction or breakout of logs to the landing in a forest or forest farm setting. It requires the ability to direct choker setters, transmit signals to the yarder operator and assist with line shifts for cable logging operations.

The unit applies to job roles for monitoring log recovery including Rigging Slinger.

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Harvesting and Haulage

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for rigging slinger operations | 1.1 Review work plan and check where required with appropriate personnel 1.2 Identify type and quantity of logs to be extracted and assess for safety and environmental protection practices 1.3 Select appropriate equipment and check for operational effectiveness in line with manufacturer’s recommendations 1.4 Plan cable operation activities in line with work order |
| 2. Plan breakout | 2.1 Plan and complete breakout operations in line with environmental conditions and workplace health and safety (WHS) procedures 2.2 Identify audible or visual signal system to be used 2.3 Assess site, rigged cable system and landing for slope, haul line, backline, lift capacity and landing chute 2.4 Identify obstacles to extraction operations and develop avoidance plan 2.5 Identify logs to be recovered with current and future haul lines 2.6 Assess information to plan extraction pattern for typical number and size of logs to be hauled 2.7 Plan line shifts during hauling; establish and prepare future rigging points and rigging |
| 3. Direct hook up of logs | 3.1 Select specific logs for next turn prior to return of rigging in line with planned operation 3.2 Assess stability of logs to be extracted and take precautions against movement in line with code of practice 3.3 Direct choker setters to prepare and hook up logs ready for extraction 3.4 Give haul signals, in line with code of practice, when all persons are clear from logs, bight of ropes and in a safe area 3.5 Monitor log movement and stop for corrective action when difficulties occur 3.6 Maintain control of movement using signalling systems until payload is in clear sight of yarder operator 3.7 Record and report breakout process and equipment faults in line with site procedures |
| 4. Assist with line shifts | 4.1. Clear all logs to be hauled with existing line position before line shift is initiated 4.2. Notch stumps in line with code requirements for the rigging method to be used 4.3. Pull slack on all lines prior to release of existing rigging 4.4. Dismantle rigging components from the existing line, check and set up on new line 4.5. Move obstructions which may cause binding or damage during hauling 4.6. Rig spars, tail trees and intermediate supports in line with site procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret: workplace health and safety and other organisational procedures work orders |
| Writing | Complete accurate reports on breakout process and equipment faults |
| Oral communication | Ask questions and actively listen to clarify contents of work plans provide clear, unambiguous verbal and non-verbal haul signals |
| Numeracy | Identify quantities of logs within work plans and complete basic calculations of time required to complete activities Complete routine calculations to determine lift capacity for rigging Complete routine calculations to plan extraction pattern for number and size of logs to be hauled |
| Interact with others | Monitor harvest team activities and provide directions to ensure efficient and safe log recovery |
| Get the work done | Identify operational, environmental and equipment problems and implement appropriate responses Select, set up, use and dismantle appropriate rigging equipment for log recovery |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPHAR3201 Monitor log recovery (rigging slinger) Release 2 | FWPHAR3201 Monitor log recovery (rigging slinger) Release 1 | Minor edits to correct system errors (wrong performance criteria for element 4) and provide clarification | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPHAR3201 Monitor log recovery (rigging slinger)

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has:

safely and efficiently monitored log recovery

In doing the above job, the individual has:

performed rigging slinger operations to recover felled logs in line with a work order and within prescribed organisational tolerances

planed the breakout process in line with site conditions

monitored log recovery using one or more of the following cable systems:

high lead (no skyline)

standing skyline

running skyline

assisted in shifting lines in line with the hook tender’s instructions.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

content and application of applicable state or territory code of practice relevant to log recovery activities

key features of varying site conditions and effects on log recovery, including:

fallen trees

general forest lean

ground growth

ground hazards and obstacles

ground slope

density of trees

wet and adverse weather conditions

wind speed and direction

environmental protection practices for forest and forest farm settings, including:

safe disposal of waste material

cleaning of plant, tools and equipment

characteristics of trees

industry standard lengths for logs

techniques used by rigging slingers in the log recovery process, including:

log extraction methods

cable logging techniques

rigging techniques

key features of varying signal systems used for log recovery activities, including:

talk-e-tooter signals

two-way radio

visual signals

whistle signals

typical industry chain of command for communication of information and instruction

purpose, features and operation of cable logging system components, including:

rigging components

cable systems: high lead (no skyline), standing skyline, running skyline

organisational procedures specific to log recovery activities

workplace health and safety with particular emphasis on use of equipment, including:

communication reporting lines

recording and reporting breakout process and equipment faults.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting

resources, equipment and materials:

cable systems

yarder and anchors

rigging equipment for log recovery

signal system equipment

personal protective equipment suitable for log recovery activities

specifications:

organisational policies and procedures relevant to monitoring log recovery

work plan for log recovery activities with specific instructions for the work site

applicable state or territory code of practice

template reports for recording breakout process and equipment faults

relationships:

a team of choker setters and yarder operators with whom the individual can interact.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47