# FWPHAR3203 Operate integrated or split flail and wood chipper with crane

Release: 1

FWPHAR3203 Operate integrated or split flail and wood chipper with crane

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to operate an integrated or split flail debarker and wood chipper with crane in an infield wood chipping operation. This unit includes preparing for infield wood chipping activities, conducting pre-operational machinery checks, setting up machinery, loading whole trees and logs with crane, monitoring machine operations, and implementing machine maintenance.

The unit applies to individuals who work as infield wood chipping machine operators in a commercial forest environment.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Harvesting and Haulage (HAR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for infield wood chipping activities | 1.1 Determine job requirements from work order or instruction and, where required, seek clarification from appropriate personnel 1.2 Confirm production targets and assess wood flow requirements to ensure efficient machine operation and overall productivity 1.3 Confirm safety and environmental protection requirements for the task according to workplace procedures 1.4 Identify, assess and take actions to mitigate risks and hazards associated with machine operations 1.5 Identify emergency procedures to follow in the event of an emergency 1.6 Identify type and quantity of material to be chipped, and quality requirements for chipped material 1.7 Plan infield wood chipping activities according to work order or instruction 1.8 Consult with appropriate personnel to ensure that work is coordinated effectively with others in the workplace 1.9 Select, fit and use personal protective equipment 1.10 Conduct pre-operational machinery checks according to manufacturer recommendations and workplace procedures |
| 2. Set up integrated or split flail and wood chipper with crane | 2.1 Determine an appropriate location for integrated or split flail and wood chipper with crane 2.2 Position machine according to work requirements, manufacturer instructions, terrain characteristics, workflow and workplace safety requirements 2.3 Secure and set up machine according to work requirements, manufacturer instructions and workplace safety requirements |
| 3. Chip whole trees and logs | 3.1 Assess whole trees and logs and dispose of or recycle material unsuitable for chipping according to environmental protection practices, quality requirements and workplace procedures 3.2 Load flail/chipper systematically using crane at feed rate applicable to safety and stability of machine, characteristics of material to be chipped, chip quality, capacity of flail and chipper and loading capacity of transport vehicles 3.3 Chip whole trees and logs to chip sizes specified in work order 3.4 Operate crane and manoeuvre log grapple safely and efficiently to avoid harm to people, flail/chipper, other machines and the environment 3.5 Confirm all merchantable product from the site is loaded into flail/chipper and all waste is managed according to work plan 3.6 Monitor output for compliance with flailing and chip specifications 3.7 Confirm safety of self and others and monitor location of people in relation to machine while machine is in operation 3.8 Load chipped material into transport vehicles according to vehicle load and distribution requirements and load covering requirements 3.9 Respond to critical situations requiring emergency shutdown to prevent personal injury or damage to machine or product |
| 4. Carry out operator maintenance | 4.1 Lock out equipment according to workplace safety and manufacturer requirements 4.2 Access tools and service equipment required for performing routine maintenance according to manufacturer instructions 4.3 Check flail chains, chipper knives, counter knives and anvil for wear and damage at appropriate or recommended intervals 4.4 Complete operator maintenance tasks according to manufacturer recommendations and workplace procedures 4.5 Record and report production outcomes and equipment faults to appropriate personnel |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace documentation to determine requirements |
| Writing | Use technical and workplace specific vocabulary to accurately and legibly complete workplace records and forms |
| Numeracy | Interpret numerical and graphical information shown on machine screens, displays and gauges relevant to safe machine operation |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPHAR3203 Operate integrated or split flail and wood chipper with crane | FWPHAR3215 Operate a heavy production mobile chipper | Redesigned unit using content from FWPHAR3215 Operate a heavy production mobile chipper to provide specific skills for specialised in-field wood chipping equipment | Not Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47