# FWPHAR3210 Conduct mechanical processor operations

Release: 2

FWPHAR3210 Conduct mechanical processor operations

# Modification History

| Release | Comment |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Application

This unit of competency describes the skills and knowledge required to operate a wheeled or tracked mechanical processor in a commercial forest harvesting environment. Activities include processing trees mechanically, segregating, and stacking logs.

The unit applies to individuals who operate a wheeled or tracked mechanical processor. Operators generally work under broad direction and take responsibility for their work.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Harvesting (HAR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Plan and prepare for operations | 1.1 Prepare to work safely according to workplace requirements 1.2 Obtain and review environmental, regulatory and legal requirements for conducting mechanical processing operations 1.3 Review work plan and clarify requirements with appropriate personnel 1.4 Select equipment appropriate to work requirements and check for operational effectiveness 1.5 Plan mechanical processor operations in line with site procedures, environmental and heritage concerns 1.6 Identify access to tree or log to be extracted and assess working conditions for safety 1.7 Identify emergency plan procedures to follow in the event of an emergency |
| 2. Operate equipment | 2.1 Conduct pre-operational checks on equipment and attachments in line with manufacturers’ recommendations and site procedures 2.2 Establish and maintain communication with other personnel to ensure safety in line with safety requirements 2.3 Detect site hazards associated with processor operations and use safe operating techniques to minimise risk 2.4 Operate mechanical processor within equipment capabilities and limitations, and manoeuvre to ensure safe and efficient operations 2.5 Adjust operating techniques to meet work requirements and site conditions |
| 3. Process individual trees | 3.1 Monitor movement and whereabouts of other personnel to ensure safety 3.2 Plan processing to ensure minimal tree damage and maximum recovery 3.3 Position equipment for stable and safe access to felled tree and to ensure loads are placed within rated load/reach capacity 3.4 Program or modify processing controller to produce standard length and diameter of logs with optimal grade and length combination in line with work plan requirements 3.5 Adjust processing techniques in response to unexpected characteristics, movement or condition of tree 3.6 Move timber/stems to pre-bunching site ensuring personnel safety and minimal damage to the timber and surrounding environment 3.7 Move equipment over pre-arranged routes with minimal damage to the environment |
| 4. Segregate and stack logs | 4.1 Locate stacks to provide appropriate space for the conduct of other landing operations in line with planned layout and site requirements 4.2 Position and construct log stacks to provide stability and minimise problems from slippage and falling of logs 4.3 Carry out maintenance of log stacks in line with planned layout and site requirements 4.4 Visually assess delivered logs and direct/move to appropriate location for further processing or stacking |
| 5. Carry out machine operator maintenance | 5.1 Follow equipment lock-out procedures in line with workplace health and safety requirements and site procedures 5.2 Conduct visual inspection and fault finding procedures in line with manufacturers’ recommendations and site procedures 5.3 Conduct routine operational servicing to maintain optimum operational performance of equipment 5.4 Diagnose faults, malfunctions or problems with equipment and report in line with site procedures 5.5 Clean and store equipment, attachments and other ancillary equipment 5.6 Record and report mechanical processor operation results in line with site procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria

| Skill | Description |
| --- | --- |
| Reading | read and understand production plans and site safety and environmental management plans in a forest harvesting environment read and understand standard operating procedures and/or manufacturers’ instructions for the operation of forest harvesting machinery |
| Writing | record operation, production and machine maintenance on paper based or electronic media |
| Oral Communication | interact verbally and non-verbally using agreed systems and processes to communicate with others on site to ensure safe and effective machine operations use appropriate vocabulary including technical language directly relevant to role |
| Numeracy | interpret gauges relevant to safe operation of harvesting machinery determine construction of stable log stack estimate the weight of logs to determine loading requirements and compliance to machine specifications |
| Navigate the world of work | understand main tasks, responsibilities and boundaries of own role |
| Interact with others | use modes of communication suitable to purpose to confirm and clarify understanding communicate and report operational and safety information to on-site personnel |
| Get the work done | recognise and respond to routine problems plan and organise own work activities and resources |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPHAR3210 Conduct mechanical processor operations Release 2 | FWPHAR3210 Conduct mechanical processor operations Release 1 | Minor additions to the foundation skills, with no other changes. | Equivalent unit |

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47