# Assessment Requirements for FWPHAR3214 Operate a single grip harvester

Release: 1

Assessment Requirements for FWPHAR3214 Operate a single grip harvester

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIHAR3214C Operate a single grip harvester, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

safely operate a single grip harvester to fell, delimb and cut trees to length

follow organisational policies and procedures for operating a single grip harvester

safely change and fit a single grip harvester cutting attachment

select appropriate cutting attachments including sprocket or bar and chain components used for felling or docking trees

communicate effectively with others in the work area

carry out routine maintenance on single grip harvester

follow applicable licensing or certification requirements.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

workplace policies, procedures and standards for operating a single grip harvester

environmental protection requirements

operational procedures and checks including start-up and shut-down procedures

equipment processes, technical capability and limitations

harvester attachments and capabilities

established communication channels and protocols to maintain safety

processes for managing risks and hazards associated with harvesting operations including:

uneven/unstable terrain

trees

fires

overhead and underground services

bridges

buildings

excavations

traffic

embankment

cuttings

structures and hazardous materials

knowledge of defects in trees that can effect processing and cause risk

workplace health and safety requirements, fire protocols and emergency evacuation procedures

procedures for recording and reporting workplace records and information.

# Assessment Conditions

The following resources must be made available:

wheeled or tracked single grip harvester and cutting attachments

site to fell and process trees

personal protective equipment required for operating a single grip harvester

work order, workplace requirements and site standards for conducting single grip harvester operations

relevant personnel for the purposes of communicating information.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume Implementation Guide - http://www.skillsimpact.com.au/forestry-and-timber/skills-standards-and-qualifications/