# FWPHAR3219 Conduct excavator operations with grabs

Release: 1

FWPHAR3219 Conduct excavator operations with grabs

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIHAR3219A Conduct excavator operations with grabs, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to operate an excavator with grabs to shift materials and complete tasks. Work is completed in a forest or forest farm setting and includes operator maintenance.

The unit applies to those who operate excavator operations with grabs in a forest environment.

Licensing, legislative, regulatory, or certification requirements apply to this unit in some states and territories at the time of publication, and may differ according to jurisdiction.

# Pre-requisite Unit

Nil

# Unit Sector

Harvesting and Haulage

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| --- | --- |
| 1. Plan excavator operations | 1.1 Review work order and where required check with appropriate personnel. 1.2 Select appropriate personal protective equipment and check for operational effectiveness. 1.3 Identify type, quantity and weight of material to be moved to determine load requirements. 1.4 Determine and confirm location, level and suitable surface to ensure stability of excavator. 1.5 Plan excavator operations in line with work order and heritage concerns. 1.6 Check excavator controls and functions, implements, brakes and manoeuvrability to ensure operational effectiveness. |
| 2. Operate excavator to shift materials | 2.1 Use equipment in line with organisational safety procedures, manufacturer’s instructions and environmental protection practices. 2.2 Position excavator on stable surface and secure to ensure stability throughout operations. 2.3 Select, attach and stabilise lifting gear in line with safe working load requirements and organisational safety procedures. 2.4 Operate excavator to shift material with optimum efficiency; observe and respond to people and hazards present. 2.5 Use communication signals throughout excavator operations to notify nearby personnel of ongoing operations. |
| 3. Complete operator maintenance | 3.1 Safely park, shut down and prepare excavator for maintenance. 3.2 Inspect excavator for faults, remove and replace defective parts in line with manufacturer’s recommendations 3.3 Complete cleaning and programmed maintenance tasks for excavator; dispose of waste materials in line with environmental protection practices. 3.4 Record and report process outcomes, equipment faults and maintenance requirements to appropriate personnel. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | identify quantities and weight of material to be moved from work orders complete routine calculations to determine material dimensions and weights for loading and shifting. |
| Oral communication skills to: | ask questions and actively listen to clarify contents of work orders provide clear, unambiguous verbal and non-verbal communication signals. |
| Reading skills to: | interpret workplace health and safety and other organisational procedures work orders manufacturers instructions and maintenance recommendations |
| Writing skills to: | complete accurate basic records for process outcomes, equipment faults and maintenance requirements. |
| Planning and organising skills to: | efficiently and logically sequence the stages of preparation and material moving, using work order to guide activities. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPIHAR3219A Conduct excavator operations with grabs

# Links

Companion Volume Implementation Guide - http://www.skillsimpact.com.au/forestry-and-timber/skills-standards-and-qualifications/