# Assessment Requirements for FWPHAR3224 Operate crawler tractor

Release: 2

Assessment Requirements for FWPHAR3224 Operate crawler tractor

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 4.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all the elements and performance criteria of this unit. There must be evidence that, on at least one occasion, the individual has:

planned, prepared and safely and effectively operated a crawler tractor to move products and/or slash in line with a production plan

checked risks associated with the operation and applied standard operating procedures

followed production plan requirements and site safety and environmental management plan

determined suitability and stability of crawler tractor for operation and safe working load limits by considering machine and equipment specifications, terrain characteristics and type and size of stems and/or logs to be moved

adjusted machine driving and positioning technique in response to terrain conditions

applied machine manipulation techniques to move products and/or slash and minimise damage to products, machine and surrounding environment in line with the production plan requirements

placed and stacked products for efficient extraction in line with the production plan requirements

communicated effectively with others in the work area

shut down, de-energised and isolated machine in line with workplace procedures and operator’s manual

carried out routine maintenance on crawler tractor parts.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

information and requirements contained in a production plan and site safety and environmental management plans

hazardous or safe working zones in forest harvesting operations

health and operational hazards or risks related to crawler tractor operations and factors that can change or introduce new hazards

industry-accepted work health and safety risk control measures, legal obligations of the current Work Health and Safety Acts or Occupational Health and Safety Acts and regulations, and requirements of industry standards and/or codes of practice for crawler tractor operations as outlined in workplace procedures and production plan

industry-accepted warning signs, their correct positioning, vehicle-to-vehicle communication systems and procedures to maintain communication and work safety during crawler tractor operations

required emergency response procedure including firefighting resources and emergency response

potential impacts of crawler tractor operations on onsite and offsite environmental values including cultural heritage, flora, fauna, ground compaction, water contamination and soil erosion

industry-accepted environmental control measures, environmental restrictions, legal obligations of the current Environment Protection Acts and regulations and requirements of industry standards and/or codes of practice for crawler tractor operations as outlined in workplace procedures and production plan

landings usage

typical location of loading and/or landing site

tree felling patterns

extraction methods

wood flow requirements and extraction and production bottlenecks particular to a production system

techniques particular to a production system to avoid mechanical damage to stems and logs during moving and stacking; position and stack logs for efficient extraction

effects of weather on ground conditions

effects of slope and terrain conditions on crawler tractor stability and operation

machine specifications, restrictions, applications and safe operating procedures, including stability and operation in different terrain conditions, slopes and road types and methods to shut down, de-energise and isolate the machine as outlined in workplace procedures or operator’s manual

using machine cabin controls

techniques for safe and efficient driving of crawler tractor

methods to conduct machine pre-operational checks and detect faults as outlined in workplace procedures or operator’s manual

location of machine’s major mechanical systems and parts of the machine in which energy is stored as outlined in operator manuals for safe and efficient conduct of the machine operator maintenance tasks

machine maintenance and servicing requirements as outlined in user and operator manuals

industry-accepted reporting requirements regarding site, incident and hazard, personal reporting, check-in, production, machine maintenance and block completion.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest harvesting site or an environment that accurately represents workplace conditions

resources, equipment and materials:

crawler tractor

suitable stems/logs and/or slash to be moved

personal protective equipment required in harvesting operations

vehicle-to-vehicle communication system

specifications:

access to workplace policies and procedures which cover current Work Health and Safety Acts or Occupation Health and Safety Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations

access to workplace policies and procedures which cover current Environment Protection Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations

access to production plan and site safety and environmental management plans

access to site emergency procedure

access to workplace standard operating procedures for single grip harvester or operator’s manual

access to user manual for vehicle-to-vehicle communication system

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47