# Assessment Requirements for FWPHAR3229 Operate single grip harvester

Release: 1

Assessment Requirements for FWPHAR3229 Operate single grip harvester

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all the elements and performance criteria of this unit. There must be evidence that, on at least one occasion, the individual has:

planned, prepared and safely and effectively operated a single gip harvester to cut and produce log products with optimal length, quality and diameter in line with log cutting instructions, quality specifications or production plan

checked risks associated with the operation and applied standard operating procedures

followed production plan requirements, log cutting instructions or quality specifications and site safety and environmental management plan

adjusted machine driving and positioning techniques in response to terrain conditions and unexpected characteristics, movement or condition of tree

applied felling and processing techniques to minimise damage to the stem, logs, standing trees, machine and surrounding environment in line with the production plan requirements

applied knowledge of log species, quality characteristics and defects during operation in line with the production plan requirements

placed, stacked and presented logs for efficient transportation and identification in line with the production plan requirements

communicated effectively with others in the work area

shut down, de-energised and isolated machine in line with workplace procedures and operator’s manual

carried out routine maintenance on single grip harvester.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

information and requirements contained in a production plan, log cutting instructions or product quality specifications and site safety and environmental management plans

hazardous or safe working zones in forest harvesting operations

health and operational hazards or risks related to single grip harvesting operations and factors that can change or introduce new hazards or risks including characteristics of difficult trees and risks associated with falling them safely

industry-accepted work health and safety risk control measures, legal obligations of the current Work Health and Safety Acts or Occupational Health and Safety Acts and regulations and requirements of industry standards and/or codes of practice for single grip harvesting operations as outlined in workplace procedures and production plan

industry-accepted warning signs, their correct positioning, vehicle-to-vehicle communication systems and procedures to maintain communication and work safety during single grip harvesting operations

required emergency response procedure including firefighting resources and emergency response

potential impacts of single grip harvester operations on onsite and offsite environmental values including cultural heritage, flora, fauna, ground compaction, water contamination and soil erosion

industry-accepted environmental control measures, restrictions, legal obligations of the current Environment Protection Acts and regulations and requirements of industry standards and/or codes of practice for single grip harvesting operations as outlined in workplace procedures and production plan

typical location of loading and/or landing site

tree felling patterns

extraction methods

wood flow requirements and extraction and production bottlenecks particular to a production system

tree shapes, species, quality characteristics and log defects particular to the site and production system

regional quality requirements for log products with particular end uses, markets and values

requirements and methods to achieve maximum stump height in response to unexpected characteristics or condition of the trees and ground level

maximum log damage and defect and minimum quality allowances particular to a production system

techniques particular to a production system to avoid mechanical damage to logs during felling, processing and stacking; achieve saw cuts at both ends of the product that meet quality specifications; avoid incorrect or excessive log butting; separate and present logs for efficient transportation and identification of the product

effects of weather on ground conditions

effects of slope and terrain conditions on machine stability and operation

machine specifications, restrictions and safe operating procedures including stability and operation in different terrain conditions, slopes and road types, chain shot rules, methods to shut down, de-energise and isolate the machine and optimal direction of travel as outlined in workplace procedures or operator’s manual

using machine cabin controls

techniques for safe and efficient driving of single grip harvester

location of machine’s major mechanical systems and parts of the machine in which energy is stored as outlined in operator manuals for safe and efficient conduct of the machine operator maintenance tasks

methods to conduct machine pre-operational checks and detect faults as outlined in workplace procedures or operator’s manual

machine maintenance and servicing requirements as outlined in workplace procedures or operator’s manual

industry-accepted reporting requirements regarding site, incident and hazard, personal reporting, check-in, production, machine maintenance and block completion.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest harvesting site or an environment that accurately represents workplace conditions

resources, equipment and materials:

single grip harvester

suitable trees to fell and process

personal protective equipment required in harvesting operations

vehicle-to-vehicle communication system

specifications:

access to workplace policies and procedures which cover current Work Health and Safety Acts or Occupation Health and Safety Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations

access to workplace policies and procedures which cover off on current Environment Protection Acts, regulations and related industry standards and codes of practice applicable to forest harvesting operations

access to production plan, site operational plan, log cutting instructions and safety and environmental management plans

access to site emergency procedure

access to workplace standard operating  procedures for single grip harvester or operator’s manual

access to user manual for vehicle-to-vehicle communication system.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47