# Assessment Requirements for FWPHAR3232 Operate harvesting machine with winch-assist system

Release: 1

Assessment Requirements for FWPHAR3232 Operate harvesting machine with winch-assist system

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

planned, prepared and safely and effectively operated two different types of harvesting machines with winch-assist system on two occasions according to a work order or work instructions. The harvesting machines must be selected from the following:

skidder

forwarder

harvester

feller buncher

carried out routine maintenance on a harvesting machine with winch-assist system on two occasions.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

information and requirements in production, site safety and environmental management plans relevant to forest harvesting operations

industry standards, codes of practice and guidelines for harvesting on steep slopes and their requirements as outlined in site safety and environmental management plans

safe work procedures and safe working zones in forest harvesting operations

hazards and risks related to operating harvesting machines with winch-assist system, and factors that can change or introduce new hazards

types and correct positioning of warning signs, vehicle-to-vehicle communication systems, and procedures to maintain communication and workplace safety during winch-assist harvesting operations

firefighting and emergency response procedures

potential impacts of winch-assist harvesting operations on onsite and offsite environmental values:

cultural heritage

flora

fauna

ground compaction

water contamination

soil erosion

environmental protection procedures relevant to forest harvesting operations

types of winch-assist systems:

systems with one cable, where the winch is attached to the harvesting machine (integral)

systems with one or more winches fitted to a machine positioned at the top of an incline (independent)

reasons for using winch-assist system:

safety

available steep slope fibre

reduce manual felling

maximum safe slope for using harvesting machines with winch assist system

features of winch-assist system:

cables and end termination fittings

safety, braking and emergency backup systems

procedures for operating winch assist system:

pre-start checks of cables and end termination fittings

operation of controls for winch-assist system

procedures for positioning machine on slope

techniques to increase traction and maximise stability of harvesting machine before and during operation

methods to detect faults as outlined in operator manual

machine and cable systems maintenance and servicing requirements as outlined in operator manual

criteria for safe placement of winch-assist machine:

access

ground stability

alignment to felling face

cable path

cable departure point from anchor

effects of slope and terrain conditions on machine stability and operation

harvesting machine types, specifications, restrictions and safe operating procedures, stability and operation in different terrain conditions and slopes, and methods to shut down the machine as outlined in workplace procedures or operator manual

reporting requirements regarding site, incident and hazard, personal reporting, check-in, production, machine maintenance and block completion.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest harvesting setting

resources, equipment and materials:

harvesting machines with winch-assist system

trees to be harvested

personal protective equipment required in harvesting operations

vehicle-to-vehicle communication system

specifications:

access to work order or instruction detailing harvesting activities to be carried out by operator

access to workplace safety and environmental protection policies and procedures applicable to forest harvesting operations

access to harvesting plan

access to site emergency procedure

access to workplace safe operating procedures or operator manual for harvesting machine and winch-assist system

access to user manual for vehicle-to-vehicle communication system.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47