# FWPHAR3232 Operate harvesting machine with winch-assist system

Release: 1

FWPHAR3232 Operate harvesting machine with winch-assist system

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to operate a harvesting machine with a winch-assist system. This unit includes planning for work, identifying hazards and risks, assessing worksite conditions, safely and securely positioning the anchor and carrying out machine operator maintenance.

The unit applies to individuals who work as winch-assisted harvester operators in a forest or plantation setting.

Individuals undertaking this unit must be competent operators of at least one of the following types of forest machines: skidder, forwarder, harvester or feller buncher.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Harvesting and Haulage (HAR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work | 1.1 Determine job requirements from work order or instruction and, where required, seek clarification from appropriate personnel 1.2 Confirm production targets for harvesting and assess wood flow requirements to ensure efficient machine operation and overall productivity 1.3 Confirm safety and environmental protection requirements for the task according to workplace procedures 1.4 Identify, assess and take actions to mitigate risks and hazards associated with winch-assist harvesting operations 1.5 Identify emergency plan procedures to follow in the event of an emergency 1.6 Identify area in which winch-assisted harvesting operations are to be conducted 1.7 Select, fit and use personal protective equipment 1.8 Inspect, test, replace and maintain cable and end termination fittings according to manufacturer specifications and workplace procedures |
| 2. Assess worksite | 2.1 Identify and assess terrain and ground conditions on worksite likely to affect winch-assisted harvesting operations 2.2 Assess maximum safe slope on worksite, taking into account terrain conditions, slope and soil type according to workplace procedures 2.3 Carry out risk assessments according to workplace procedures 2.4 Evaluate outcomes of assessment of site conditions, maximum safe slope and risks to determine if any changes are required to work order or work instructions |
| 3. Position winch-assist machine | 3.1 Determine position of winch assist machine according to workplace procedures 3.2 Assess proposed position of winch assist machine against criteria for placement of machine according to safety and workplace procedures 3.3 Test position of winch assist machine according to workplace procedures |
| 4. Operate winch-assist harvesting machine | 4.1 Connect harvesting machine to winch assist machine according to manufacturer instructions and workplace procedures 4.2 Carry out pre-start checks of harvesting machine according to manufacturer instructions and workplace procedures 4.3 Position harvesting machine on slope according to workplace procedures 4.4 Check and maintain condition of cable by setting correct tension, working within specified departure angles, minimising shock loading, inspecting for wear and integrity, and avoiding physical damage to cable 4.5 Apply techniques to increase traction and maximise stability of harvesting machine before and during commencing operational tasks 4.6 Complete operational tasks according to work order or work instructions |
| 5. Carry out machine operator maintenance | 5.1 Park and shut down harvester according to manufacturer instructions and workplace procedures 5.2 Clean, check and carry out routine maintenance activities on harvesting machine and winch-assist machine to maintain optimum operational performance according to manufacturer specifications 5.3 Identify and report faults, malfunctions or problems according to workplace procedures and manufacturer recommendations 5.4 Record harvester operations and maintenance activities |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace documentation to determine work requirements |
| Writing | Use technical and enterprise specific vocabulary to accurately and legibly complete workplace records and forms |
| Oral communication | Ask questions to clarify contents of production plans |
| Numeracy | Interpret numerical and graphical information shown on machine screens, displays and gauges relevant to safe machine operation |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPHAR3232 Operate harvesting machine with winch-assist system | Not applicable | This unit has been created to address an emerging skill or task required by industry | Newly created |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPHAR3232 Operate harvesting machine with winch-assist system

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

planned, prepared and safely and effectively operated two different types of harvesting machines with winch-assist system on two occasions according to a work order or work instructions. The harvesting machines must be selected from the following:

skidder

forwarder

harvester

feller buncher

carried out routine maintenance on a harvesting machine with winch-assist system on two occasions.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

information and requirements in production, site safety and environmental management plans relevant to forest harvesting operations

industry standards, codes of practice and guidelines for harvesting on steep slopes and their requirements as outlined in site safety and environmental management plans

safe work procedures and safe working zones in forest harvesting operations

hazards and risks related to operating harvesting machines with winch-assist system, and factors that can change or introduce new hazards

types and correct positioning of warning signs, vehicle-to-vehicle communication systems, and procedures to maintain communication and workplace safety during winch-assist harvesting operations

firefighting and emergency response procedures

potential impacts of winch-assist harvesting operations on onsite and offsite environmental values:

cultural heritage

flora

fauna

ground compaction

water contamination

soil erosion

environmental protection procedures relevant to forest harvesting operations

types of winch-assist systems:

systems with one cable, where the winch is attached to the harvesting machine (integral)

systems with one or more winches fitted to a machine positioned at the top of an incline (independent)

reasons for using winch-assist system:

safety

available steep slope fibre

reduce manual felling

maximum safe slope for using harvesting machines with winch assist system

features of winch-assist system:

cables and end termination fittings

safety, braking and emergency backup systems

procedures for operating winch assist system:

pre-start checks of cables and end termination fittings

operation of controls for winch-assist system

procedures for positioning machine on slope

techniques to increase traction and maximise stability of harvesting machine before and during operation

methods to detect faults as outlined in operator manual

machine and cable systems maintenance and servicing requirements as outlined in operator manual

criteria for safe placement of winch-assist machine:

access

ground stability

alignment to felling face

cable path

cable departure point from anchor

effects of slope and terrain conditions on machine stability and operation

harvesting machine types, specifications, restrictions and safe operating procedures, stability and operation in different terrain conditions and slopes, and methods to shut down the machine as outlined in workplace procedures or operator manual

reporting requirements regarding site, incident and hazard, personal reporting, check-in, production, machine maintenance and block completion.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest harvesting setting

resources, equipment and materials:

harvesting machines with winch-assist system

trees to be harvested

personal protective equipment required in harvesting operations

vehicle-to-vehicle communication system

specifications:

access to work order or instruction detailing harvesting activities to be carried out by operator

access to workplace safety and environmental protection policies and procedures applicable to forest harvesting operations

access to harvesting plan

access to site emergency procedure

access to workplace safe operating procedures or operator manual for harvesting machine and winch-assist system

access to user manual for vehicle-to-vehicle communication system.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47