# FWPNSY3007 Operate automated topping system as part of tree production nursery operations

Release: 1

FWPNSY3007 Operate automated topping system as part of tree production nursery operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Application

This unit of competency describes the skills and knowledge required to operate an automated topping system as part of tree production nursery operations.

The unit applies to individuals who operate automated topping systems as part of tree production nursery operations.

All work must be carried out to comply with workplace procedures according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Forest Nursery Production (NSY)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work | 1.1 Determine job requirements from work order or instruction, and where required seek clarification from appropriate personnel 1.2 Confirm safety and environmental requirements, including use of personal protective equipment and machine guards, for the task according to workplace procedures 1.3 Identify, assess and take actions to mitigate risks and hazards associated with working with automated topping system 1.4 Consult with team members to ensure that automated topping system operations are coordinated effectively 1.5 Select seedlings to be trimmed according to job requirements 1.6 Confirm production capacity and full range of functions of automated topping system |
| 2. Set up topping system | 2.1 Check existence and serviceability of safety equipment, including emergency stops, gauges, guards and controls 2.2 Select topping system program and make adjustments according to job requirements and manufacturer instructions 2.3 Load and/or position seedlings according to manufacturer instructions and production requirements 2.4 Conduct trial runs to check topping operation, accuracy, alignment and that finished product meets quality requirements 2.5 Make necessary adjustments to topping system settings based on outcomes of trial runs |
| 3. Operate topping system | 3.1 Complete automated topping system start-up procedure according to manufacturer instructions 3.2 Operate automated topping system according to designed capacity, purpose and manufacturer recommendations 3.3 Monitor topping system operation to ensure product quality and output 3.4 Minimise waste according to workplace and environmental protection procedures 3.5 Recycle, reuse or dispose of cuttings according to workplace procedures 3.6 Identify problems with the required work and/or the operation of the automated topping system, and rectify or report to appropriate persons 3.7 Shut topping system down according to workplace procedures 3.8 Record production outcomes and equipment faults or malfunctions and corrective actions taken according to workplace procedures |
| 4. Maintain topping equipment | 4.1 Perform routine maintenance according to purpose and manufacturer recommendations 4.2 Tag and report faulty and/or defective equipment according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret complex written, technical information in operating manuals to confirm equipment operations and perform troubleshooting |
| Writing | Complete routine workplace records related to equipment faults and product quality |
| Oral communication | Use active listening and questioning techniques to convey and clarify information |
| Numeracy | Perform routine calculations related to time, dimensions, production capacity, volume and number |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPNSY3007 Operate automated topping system as part of tree production nursery operations | Not applicable | The unit has been created to address a skill or task required by industry that is not covered by an existing unit | Newly created |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPNSY3007 Operate automated topping system as part of tree production nursery operations

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with Forest and Wood Products Training Package Version 7.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated and maintained an automated topping system as part of tree production nursery operations for one batch in one production run.

In undertaking this activity, the individual has:

determined job requirements

confirmed safety and environmental requirements, including use of personal protective equipment (PPE) and emergency stops, gauges, guards and controls

selected seedlings to be trimmed according to job requirements

confirmed production capacity and full range of functions of automated topping system

selected topping system program and loaded and/or positioned seedlings

conducted trial runs to check topping operation, accuracy, alignment and that finished product met quality requirements

operated and monitored topping system

recorded production outcomes and equipment faults or malfunctions and corrective actions taken

performed routine operator maintenance on topping machine system.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

types, functions, components, accessories and capacity of automated topping systems used in tree production nursery operations

types of blades:

reciprocating

rotating

sources of information on automated topping system operation and maintenance

reasons for using automated topping systems in tree production nursery operations:

speed

controlling accuracy

consistency of product dimensions

reducing mechanical injury

methods for:

setting up automated topping system

selecting programs for automated topping system

operating automated topping system

quality assurance

troubleshooting automated topping system

maintaining automated topping system

maintaining and/or replacing blades

reporting machinery faults and material defect

basic components and functions of mechatronic systems:

sensors

actuators

controllers

mechanical components

environmental sustainability issues as they relate to the work task

workplace health and safety requirements specific to the work task.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest environment or an environment that accurately represents workplace conditions

resources, equipment and materials:

an automated topping system for forestry nursery production

commercial quantities of seedlings to be topped

PPE suitable for operation of automated topping system

relationships:

work team for seed sowing line

specifications:

workplace procedures for setting up, operating and maintaining automated topping systems

workplace health, safety and environmental policies and procedures applicable to forestry nursery production.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47