# Assessment Requirements for FWPSAW2211 Round softwood logs

Release: 1

Assessment Requirements for FWPSAW2211 Round softwood logs

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has, on at least on one occasion, rounded softwood logs using a machine that shapes the debarked log into a round pole, and has:

followed work order and relevant workplace health and safety and environmental protection procedures

identified, assessed and taken actions to mitigate risks and hazards associated with rounding activities

rounded softwood logs following workplace policies and procedures

stored poles according to workplace requirements

completed each of the following basic non-specialist maintenance activities on at least one occasion:

checked the operation of emergency stop buttons and lock-out systems

cleaned the machine of wood chips and sawdust

replaced blunt cutters

inspected machine parts for wear or damage, including guards

greased and lubricated moving parts

completed handover functions.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose, features and operation of equipment used to round softwood logs:

measuring equipment

rounding plant

scanners that calibrate dimension sizes

back-up computer device

automated conveyor system to transport logs to and from the rounding plant

maintenance tools and equipment for cutters

methods used to assess logs for:

diameter

curvature

faults

species

moisture content

typical work flow processes for rounding logs

typical flow of log supply in and out of the production area

range of feed rates and how they affect production output and finished quality of rounded logs

methods for assessing and rectifying typical scanner problems:

tangles

sorting pockets

monitor failure

methods for assessing cutter condition

log rounding systems, processes, feed rates and finish

characteristics of timber

storage systems and labelling

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

typical log defects and how they affect ability to produce quality rounded softwood logs:

warp

wane

curvature

cupping

shakes

insect defects

knots

resin pockets

workplace procedures specific to rounding logs:

workplace health and safety, with particular emphasis on equipment lock-out and use of personal protective equipment (PPE)

communication reporting lines

recording and reporting processing problems and equipment faults.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

measuring equipment

moisture probes

rounding plant

automated conveyor system to transport logs to and from the rounding plant

maintenance tools and equipment for cutters

consumable cutters

PPE suitable for producing sawing and moving flitches and cants

a selection of logs

specifications:

manufacturer instructions for use, repair and maintenance of equipment

work order with specific instructions for rounding logs and despatch to downstream processing

template documents for recording equipment faults and maintenance requirements

workplace procedures for rounding logs.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47