# Assessment Requirements for FWPSAW3204 Saw flitches and cants

Release: 2

Assessment Requirements for FWPSAW3204 Saw flitches and cants

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to sawing flitches and cants

saw log sections to produce flitches and cants in line with a work order and within prescribed organisational specifications

complete these types of basic non-specialist maintenance:

cleaning the machine of wood chips and sawdust

replacing blunt saw blades

inspecting machine parts for wear or damage, including guards

greasing and lubricating moving parts

checking the operation of emergency stop buttons and lock out systems.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

purpose, features and operation of equipment used to saw flitches and cants:

carriages - standard three headed dog carriages and flat top carriages with head rigs

saws - circular saws, bench saws  and overhead frame saws or band saws

mechanical feed

conveyor system to transport logs to and from the sawing area

maintenance tools and equipment for saws

typical work flow processes for sawing flitches and cants

typical flow of log supply in and out of the sawing area

range of cutting patterns used on logs to achieve maximum output of flitches and cants

range of feed rates and how they affect production output and finished quality of flitches and cants

methods for assessing saw blade condition

characteristics of timber

storage systems and labelling

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

typical log defects and how they affect ability to produce quality flitches and cants:

warp

wane

curvature

cupping

shakes

insect defects

knots

resin pockets

organisational policies and procedures specific to sawing flitches and cants:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting processing problems and equipment faults.

# Assessment Conditions

The following resources must be made available:

one or more of the following types of carriages:

standard three headed dog carriages

flat top carriages with head rigs

one or more of the following types of saws:

circular saws

bench saws

overhead frame saws or band saws

mechanical feed

conveyor system to transport logs to and from the sawing area

maintenance tools and equipment for saws

consumable saw blades

personal protective equipment suitable for rounding logs

a selection of logs

manufacturers’:

instructions for equipment use

recommendations for repairing and maintaining equipment

work order with specific instructions for sawing flitches and cants and despatch to downstream processing

template documents for recording equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational policies and procedures for sawing flitches and cants.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47