# Assessment Requirements for FWPSAW3210 File and set saws

Release: 2

Assessment Requirements for FWPSAW3210 File and set saws

# Modification History

| Release | Comments |
| --- | --- |
| Release 2 | This version released with FWP Forest and Wood Products Training Package Version 3.0. |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 2.0. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to filing and setting saws

bevelling teeth and assessing bevel lengths, widths, angles and surface texture, in line with standard operating procedures and site production requirements

removing and replacing saws from equipment

setting teeth and assessing conditions in line with standard operating procedures

complete these assessments on saw blade condition:

cracks and movement in existing welding repairs

altered temper and hardness of blade from excessive heat during cutting

weaknesses or damage caused through stress of bending teeth.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

purpose, features and operation of saws for which blades are filed and set:

hand saws

band saws

circular saws

gang saws

features and names of different parts of saw blades

purpose, features and operation of tools and equipment used to file and set saw blades:

jigs

vices

setting instruments; levers and gauges

automatic or hand-held grinders

bevel files

importance of setting teeth to match requirements for:

characteristics of various materials sawn

speed of the sawing process

direction of the sawing process

cutting angle for sawing

saw blade specifications

methods for:

removing saw blades from saws

assessing saw blade condition

setting teeth to a specified profile

grinding and bevelling teeth to required shape, tolerance and pitch

compensating for the effects of spring back

tooth pitch and tooth angles at which blades are bevelled (blade geometry)

characteristics of metal used in saw blades; causes and consequences of spring back

organisational policies and procedures specific to filing and setting saw blades:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting processing and equipment faults

environmental protection practices for repairing and maintaining tools and equipment in forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing materials.

# Assessment Conditions

The following resources must be made available:

tools and equipment used to file and set saw blades:

jigs

vices

setting instruments; levers and gauges

automatic or hand-held grinders

bevel files

one or more of the following types of saws to be repaired and maintained:

hand saws

band saws

circular saws

gang saws

personal protective equipment suitable for filing and setting saw blades

manufacturers’ instructions for use, repair and maintenance of equipment

work order for repair and maintenance activities with specific instructions for filing and setting saw blades in specified equipment

relevant personnel for the purposes of communicating information

template documents for recording processing and equipment faults

organisational policies and procedures for filing and setting saw blades.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47