# Assessment Requirements for FWPSAW3223 Assess wood chips

Release: 1

Assessment Requirements for FWPSAW3223 Assess wood chips

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPISAW3223B Assess wood chips, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to assessing wood chips

assess wood chips in line with a work order and within prescribed organizational requirements for the following:

size distribution

shape

finish quality

contamination

faults

safely use tools, machinery and equipment for assessing wood chips

identify the following contaminants in wood chips:

disease

infestation

rot

paint

plastic

bark

charcoal

rock

metal

measure moisture in wood chips

identify whether wood chips meet requirements for size and shape

record sampling processes and outcomes of wood chip assessment.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

how wood chips are produced, transported and stored

environmental protection requirements for the safe disposal of waste or contaminated wood chips

organisational and site standards, requirements, policies and procedures for assessing wood chips

environmental risks and hazards

sampling and testing techniques

types and characteristics of different wood chips

visual assessment techniques

storage systems and labelling

tools and equipment and their safe use, operation and maintenance during the assessment of wood chips

procedures for recording and reporting workplace information.

# Assessment Conditions

The following resources must be made available:

wood chips

equipment for measuring moisture in wood chips

personal protective equipment

relevant personnel for the purposes of communicating information

documents, spreadsheets and commonly used proforma documents

organisational policies and procedures relevant to assessing wood chips.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47