# Assessment Requirements for FWPSAW3226 Saw logs using CNC optimising systems

Release: 1

Assessment Requirements for FWPSAW3226 Saw logs using CNC optimising systems

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPISAW3226B Saw logs using CNC optimising systems, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to sawing logs using CNC optimising systems

saw logs using CNC optimising systems in line with work order and within prescribed organisational tolerances

set computer programs for the CNC optimising system to follow

complete these types of basic non-specialist maintenance:

cleaning the machine of wood chips and sawdust

replacing blunt saw blades

inspecting machine parts for wear or damage, including guards

greasing and lubricating moving parts

checking the operation of emergency stop buttons and lock out systems.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

reducing water and energy use

typical log defects and how they affect ability to produce quality flitches and cants:

warp

wane

curvature

cupping

shakes

insect defects

knots

resin pockets

typical work flow processes for sawing flitches and cants with CNC optimising saws

typical flow of log supply in and out of the sawing area

range of cutting patterns used on logs to achieve maximum output of flitches and cants

range of feed rates and how they affect production output and finished quality of flitches and cants

purpose, features and operation of CNC optimising and other equipment used to saw flitches and cants:

CNC optimising saws

CNC computer program

mechanical feed

conveyor system to transport logs to and from the sawing area

maintenance tools and equipment for saws

methods for assessing saw blade condition

organisational procedures specific to operating CNC optimising saws:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting processing problems and equipment faults.

# Assessment Conditions

The following resources must be made available:

CNC optimising saws

CNC computer program

maintenance tools and equipment for saws

consumable saw blades

personal protective equipment suitable for operating CNC optimising saws

a diverse, comprehensive and commercial range of logs

manufacturers’:

instructions for equipment use

recommendations for repairing and maintaining equipment

work order with specific instructions for sawing logs and despatch to downstream processing

template documents for recording equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational procedures for operating CNC optimising saws.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47