# Assessment Requirements for FWPSAW3227 Select and saw logs in multi-species operations

Release: 1

Assessment Requirements for FWPSAW3227 Select and saw logs in multi-species operations

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPISAW3227B Select and saw logs in multi-species operations, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and procedures relevant to selecting and sawing logs in multi-species operations

select logs in line with production requirements and log utilisation, including assessment of

diameter range

curvature

faults

softwood species of various types

moisture content

select and saw logs in multi-species operations to produce flitches in line with a work order and within prescribed organisational specifications

adjust break down process regularly to allow for changes in species

complete these types of basic non-specialist maintenance:

cleaning the machine of wood chips and sawdust

replacing blunt saw blades

inspecting machine parts for wear or damage, including guards

greasing and lubricating moving parts

checking the operation of emergency stop buttons and lock out systems.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

purpose, features and operation of equipment used to saw flitches and cants:

carriages:

standard three headed dog carriages

flat top carriages with head rigs

saws:

circular saws

bench saws

overhead frame saws or band saws

mechanical feed

conveyor system to transport logs to and from the sawing area

maintenance tools and equipment for saws

typical range of industry and organisation specific species types:

visual characteristics of logs

recovery characteristics

typical work flow processes for sawing flitches and cants

methods of adjusting workflow and sawing schedule during operation to account for differences in species

range of cutting patterns used on logs to achieve maximum output of flitches and cants

different cutting patterns required for different species of trees

assessment of logs for:

diameter range

curvature

faults

softwood species of various types

moisture content

range of feed rates and how they affect production output and finished quality of flitches and cants

typical flow of log supply in and out of the sawing area

methods for assessing saw blade condition

species and recovery characteristics of timber and timber defects

cutting pattern requirements, feed rates and finish

flow control systems and marking requirements

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

typical log defects and how they affect ability to produce quality flitches and cants:

warp

wane

curvature

cupping

shakes

insect defects

knots

resin pockets

organisational procedures specific to selecting and sawing logs in multi-species operations:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting processing problems and equipment faults.

# Assessment Conditions

The following resources must be made available:

one or more of the following types of carriages:

standard three headed dog carriages

flat top carriages with head rigs

one or more of the following types of saws:

circular saws

bench saws

overhead frame saws or band saws

mechanical feed

conveyor system to transport logs to and from the sawing area

maintenance tools and equipment for saws

consumable saw blades

personal protective equipment suitable for selecting and sawing logs

a selection of logs from different species of trees

manufacturers’:

instructions for equipment use

recommendations for repairing and maintaining equipment

work order with specific instructions for selecting and sawing logs and despatch to downstream processing

template documents for recording equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational procedures for selecting and sawing logs in multi-species operations.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47