# Assessment Requirements for FWPSAW3247 Replace tungsten tips

Release: 1

Assessment Requirements for FWPSAW3247 Replace tungsten tips

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 5.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has:

followed work order and relevant workplace health and safety and environmental protection procedures

removed worn tungsten tips from circular saw blades

replaced tungsten tips for circular saw blades

assessed condition of tungsten tips in line with workplace procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

basic knowledge of physics of sawing, including:

saw blade material and properties

impact of timber properties on saw blades

direction of cut

forces on a blade

heat removal

saw tooth configuration

purpose, features and operation of saws for which tungsten tips are replaced:

circular saws

features and names of different parts of saw blades

purpose, features and operation of tools and equipment used to replace tungsten tips:

brazing equipment and brazing material

brazing jigs

methods for:

removing saw blades from saws

assessing saw blade condition

removing tungsten tips from saw blades

cleaning excess braze alloy and carbide fragments from tip seats

lengthening or deepening tip seats to match replacement tip requirements

selecting size and type of replacement tip

pre-tinning new tips

brazing new tungsten tips onto the tooth face or tooth seat

appropriate tolerances for brazing alloy joint thickness, replacement tip sizes and side clearance objectives

characteristics of metal used in saw blades, and effects of brazing

types of risk and hazards and mitigation measures associated with replacing tungsten tips

workplace procedures specific to replacing tungsten tips:

workplace health and safety, with particular emphasis on equipment lock-out and use of personal protective equipment (PPE)

communication reporting lines

recording and reporting processing and equipment faults

environmental protection practices applicable to saw technician workshops or timber processing facilities:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing materials.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a saw technician workshop, timber processing facility or an environment that accurately represents workplace conditions

resources, equipment and materials:

brazing equipment

brazing jigs

circular saws

tungsten tips

brazing material: silver alloy, flux in the form of a liquid or paste

side clearance dial indicator: Kafer Gauge

PPE suitable for replacing tungsten tips

specifications:

documents, spreadsheets and templates for recording operation and equipment faults

manufacturer instructions for use, repair and maintenance of equipment

work order with specific instructions for replacing tungsten tips for circular saws

workplace procedures for replacing tungsten tips.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47