# FWPSAW3252 Assemble, operate and dismantle a portable sawmill

Release: 1

FWPSAW3252 Assemble, operate and dismantle a portable sawmill

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Application

This unit of competency describes the skills and knowledge required to assemble, operate and dismantle a portable sawmill.

The unit applies to individuals who assemble, operate and dismantle portable sawmills.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Sawmilling and Processing (SAW)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Assemble portable sawmill | 1.1 Determine job requirements from work order or instructions and, where required, confirm with appropriate personnel 1.2 Confirm workplace health and safety and environmental protection practices and/or procedures for task, including use of personal protective equipment, equipment lock-out and safe manual handling 1.3 Identify and assess hazards and take actions to mitigate risks related to assembling, operating and dismantling portable sawmills 1.4 Inspect suitability of proposed worksite and identify potential workflow interferences 1.5 Identify safe location for equipment, logs, stacking of sawn timber and waste 1.6 Assemble portable sawmill on site according to manufacturer instructions 1.7 Complete pre-operational checks on portable sawmill 1.8 Conduct test run of portable sawmill and resolve or report processing and equipment faults |
| 2. Saw logs using portable sawmill | 2.1 Use cant hooks, winches or log rollers to position logs to be sawn and stabilise with chock logs or pre-notched runners 2.2 Determine sawn timber dimensions from customer order or work order 2.3 Identify log characteristics and estimate number of timber boards and sequence of cuts from an individual log 2.4 Adjust saw, carriage and mill to suit dimensions of boards to be cut 2.5 Operate portable sawmill, ensuring that sawn edges are controlled 2.6 Remove logs with defects, reject boards and off-cuts to waste or sawn recovery 2.7 Record and report process and equipment faults |
| 3. Conduct operator maintenance | 3.1 Lock out and isolate portable sawmill according to workplace health and safety procedures 3.2 Check saw and saw blades for bluntness or damage, and repair or replace according to manufacturer instructions |
| 4. Dismantle portable sawmill | 4.1 Dismantle portable sawmill according to manufacturer instructions 4.2 Load and secure dismantled sawmill equipment for transportation 4.3 Restore work site according to workplace environmental protection practices and/or procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret routine workplace documents relevant to portable sawmilling operations |
| Writing | Complete routine workplace records used in sawmilling operations |
| Oral communication | Ask open and closed questions and actively listen to clarify contents of work orders |
| Numeracy | Interpret routine numerical information in work orders Perform routine calculations or estimations of number and dimension relevant to sawmilling operations |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPSAW3252 Assemble, operate and dismantle a portable sawmill | FWPSAW2209 Dismantle, transport and assemble hand portable sawmill | Redesigned unit that includes content from FWPSAW2209 Dismantle, transport and assemble hand portable sawmill and FWPSAW3229 Operate a portable sawmill | Not equivalent |
| FWPSAW3252 Assemble, operate and dismantle a portable sawmill | FWPSAW3229 Operate a portable sawmill | Redesigned unit that includes content from FWPSAW2209 Dismantle, transport and assemble hand portable sawmill and FWPSAW3229 Operate a portable sawmill | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPSAW3252 Assemble, operate and dismantle a portable sawmill

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

selected an appropriate site and assembled one portable sawmill following workplace procedures

selected logs and operated one portable sawmill for one production period to produce green timber boards according to a work order

performed routine operator maintenance on one portable sawmill at the completion of a production period

disassembled and prepared one portable sawmill for transportation.

In performing each of these tasks, the individual has:

followed workplace operating and health and safety procedures, including use of personal protective equipment (PPE), equipment lock-out and safe manual handling

identified and controlled hazards associated with the work task

followed workplace environmental protection practices and/or procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

features and safe use of portable sawmills and other equipment, including:

cant hooks

winches

log rollers

mechanical lifting equipment

appropriate locations for portable sawmills to optimise efficient and safe workflow

pre-start-up checks for portable sawmills, including:

ensuring that saw has been set up correctly

ensuring that blades are installed accurately

ensuring that machinery is operating to optimum performance

optimisation of log sawing, including:

planning the cutting of a log to achieve the most from it (most and widest boards possible)

ensuring environmental protection requirements and practices are central to this planning process

characteristics of logs for sawing, including:

diameter range

curvature

faults

species

moisture content

log sawing processes, feed rates and finish

blade condition assessment

log defects, including:

defects arising from a change in moisture content

warp

wane

curvature

shakes

insect defects

knots and resin pockets

sweep

taper

end splits

shape

non-specialist maintenance for portable sawmills, including:

cleaning machine of wood chips and sawdust

replacing blunt and damaged saw blades

inspecting machine parts for wear or damage, including guards

greasing and lubricating moving parts

checking operation of emergency stop buttons and lock-out systems

workplace procedures for assembling, operating and disassembling a portable sawmill, including:

health and safety, including use of PPE, equipment lock-out and safe manual handling

portable sawmill operation

communication reporting lines

recording and reporting processing and equipment faults

hazards and associated risks with assembling, operating and dismantling a portable sawmill

workplace environmental protection requirements and procedures for safe disposal of waste material and cleaning of plant, tools and equipment.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a forest or timber processing workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

portable sawmill

tools and equipment required for assembling, operating and dismantling a portable sawmill

PPE suitable for assembling, operating and dismantling a portable sawmill

quantities of logs

specifications:

work order or instruction detailing activities to be carried out

workplace health and safety and environmental protection policies and/or procedures applicable to assembling, operating and dismantling a portable sawmill

workplace procedures for assembling, operating and dismantling a portable sawmill.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47