# FWPSAW3255 Operate conventional log breakdown saw line

Release: 1

FWPSAW3255 Operate conventional log breakdown saw line

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Application

This unit of competency describes the skills and knowledge required to operate a conventional breakdown saw line to saw logs as part of primary and secondary processing or green mill operations in a sawmill.

The unit applies to individuals who saw logs using conventional log breakdown saw lines in a sawmill or timber processing work environment.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Sawmilling and Processing (SAW)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for primary or secondary log breakdown | 1.1 Determine job requirements from work order or instructions and, where required, check with appropriate personnel 1.2 Review workplace health and safety and environmental protection practices and/or procedures for work activity, including emergency procedures to be followed in the event of an emergency situation 1.3 Identify and assess hazards and take actions to mitigate risks related to performing log breakdown 1.4 Check production input to ensure that it meets job specifications 1.5 Check that other upstream and downstream processing stages are ready for production 1.6 Select cutting schedule based on incoming production input and job requirements to optimise grade, size and recovery 1.7 Conduct trial runs to check for quality and compliance with cutting schedule requirements 1.8 Identify processing constraints and potential solutions according to workplace procedures |
| 2. Operate primary or secondary log breakdown equipment | 2.1 Complete pre-start-up checks according to manufacturer and workplace procedures 2.2 Operate breakdown equipment to comply with selected cutting schedule 2.3 Manage product flow to ensure efficient workflow in sawmill and breakdown operations |
| 3. Monitor performance of breakdown equipment | 3.1 Monitor and interpret output data from breakdown equipment control system according to workplace procedures 3.2 Monitor and assess output quality from breakdown equipment, identify equipment faults and other sources of quality variations, and take corrective action or report to appropriate personnel according to workplace procedures 3.3 Complete production, maintenance and quality records according to workplace procedures |
| 4. Implement routine maintenance procedures for breakdown equipment | 4.1 Carry out preventative maintenance and cleaning for breakdown equipment according to workplace procedures 4.2 Coordinate saw changes with other staff, and make adjustments to breakdown equipment according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Writing | Prepare routine written reports using required format, language and structure |
| Oral communication | Employ active listening and questioning on routine matters related to output quality and equipment history records |
| Numeracy | Count number of logs to be sawn Perform basic calculations to determine feed rate to optimise quality and quantity of production output |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPSAW3255 Operate conventional log breakdown saw line | FWPSAW3230 Perform primary log breakdown | Redesigned unit that includes content from FWPSAW3230 Perform primary log breakdown, FWPSAW3231 Perform secondary log breakdown and FWPSAW3241 Set up and operate production saws used in sawmill operations | Not equivalent |
| FWPSAW3255 Operate conventional log breakdown saw line | FWPSAW3231 Perform secondary log breakdown | Redesigned unit that includes content from FWPSAW3230 Perform primary log breakdown, FWPSAW3231 Perform secondary log breakdown and FWPSAW3241 Set up and operate production saws used in sawmill operations | Not equivalent |
| FWPSAW3255 Operate conventional log breakdown saw line | FWPSAW3241 Set up and operate production saws used in sawmill operations | Redesigned unit that includes content from FWPSAW3230 Perform primary log breakdown, FWPSAW3231 Perform secondary log breakdown and FWPSAW3241 Set up and operate production saws used in sawmill operations | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPSAW3255 Operate conventional log breakdown saw line

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has run and monitored a conventional log breakdown saw line for one production cycle in an equipment configuration specific to one of the following sawmill operations:

primary processing operations

secondary processing operations.

In performing this task, the individual has:

followed workplace health and safety and operating procedures

checked equipment, production input and process readiness to ensure operational effectiveness

selected and adhered to a cutting schedule during breakdown operation to optimise grade, size and recovery

verified output quality and identified two real or hypothetical reasons for quality variations and corrective actions

performed routine equipment maintenance and cleaning operations

produced routine reports containing information on production, maintenance and quality.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

types of hazards, associated risks and mitigation measures associated with breaking down logs

workplace procedures specific to breaking down logs, including:

health and safety, including use of personal protective equipment (PPE) and equipment lock-out

communication reporting lines

recording and reporting of processing problems and equipment faults

emergency procedures

workplace environmental protection requirements and practices specific to timber processing plants, including:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

log breakdown equipment in workplace environment, including:

configuration

purpose

components

operating parameters

capability

operation

safety features of breakdown equipment, including:

isolation procedures

stop button

lock-out switch or system

hold cards

guards

log breakdown process

distinction between primary and secondary log breakdown

typical workflow for breaking down logs

downstream processing options for log breakdown operations

range of cutting patterns used on logs to optimise for maximum recovery and maximum value

range of feed rates and how they affect production output and finished quality

factors that impact on output quality in log breakdown operations, including:

log or timber characteristics and defects

variations in size

blunt and damaged saws

log breakdown equipment faults, including methods for assessing saw blade condition

routine operator maintenance activities associated with sawing operations.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber processing plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

conventional log breakdown equipment

PPE suitable for log breakdown operations

logs for processing

specifications:

workplace procedures for log breakdown operations

workplace emergency procedures relevant to log breakdown operations

workplace health and safety and environmental protection policies and/or procedures applicable to log breakdown operations.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47