# FWPTMM3201 Convert timber

Release: 1

FWPTMM3201 Convert timber

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPITMM3201B Convert timber, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. |

# Application

This unit of competency describes the outcomes required to prepare and re-saw material to width and on the thickness with a power driven saw to optimise timber conversion and usability. Work includes non-specialist saw maintenance and is completed in a forest and wood products factory setting.

The unit applies to Sawyer, Saw Operator, Machine Operator, Timber Products Technician.

No licensing, legislative, regulatory, or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Timber Manufactured Products

Timber Merchandising

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |
| --- | --- |
| 1. Prepare for conversion | 1.1 Review work order and where required check with appropriate personnel. 1.2 Obtain type and quantity of material to be converted from storage location. 1.3 Select appropriate equipment and check for operational effectiveness in line with manufacturer’s recommendations. 1.4 Plan timber conversion and saw set-up in line with work plan. |
| 2. Produce boards | 2.1 Set and regularly check saw, carriage and feed and adjust to suit conversion plan, ordered sizes and tolerances. 2.2 Use equipment and machinery in line with organisational procedures, manufacturer’s instructions and environmental protection practices. 2.3 Monitor timber supply to ensure constant supply to saw. 2.4 Maintain feed rate and operate saw bench to produce boards without damage to boards or saw blade. 2.5 Dispose of incorrect cuts, off-cuts and material with defects in line with environmental protection practices. 2.6 Stack material safely in designated location in line with work order requirements. 2.7 Report processing and equipment faults to appropriate personnel. |
| 3. Complete operator maintenance | 3.1 Follow organisational safety procedures to lock-out equipment. 3.2 Check saw blades for bluntness or damage. 3.3 Remove and replace saw blades in line with manufacturer’s recommendations. 3.4 Dispose of used blades in line with organisational procedures and environmental protection practices. 3.5 Keep saw area clear of dust, off-cuts and debris in line with WHS and site requirements. |

# Foundation Skills

| This section describes those core and employment skills that are essential to performance and are not explicit in the performance criteria. |  |
| --- | --- |
| Numeracy skills to: | identify quantities of required material within work plans and count raw materials interpret order sizes and set up the saw to achieve correct depth, width and thickness of cut calculate the feed rate to optimise quality and quantity of production output. |
| Oral communication skills to: | ask questions and actively listen to clarify contents of work orders make accurate verbal reports on processing and equipment faults. |
| Reading skills to: | interpret: WHS and other organisational procedures work orders manufacturers’ instructions and maintenance recommendations. |
| Planning and organising skills to: | efficiently and logically sequence the stages of preparation and production using work order to guide activities. |
| Problem-solving skills to: | recognise faulty equipment and resolve by reporting or rectifying. |
| Technology skills to: | select, use and maintain appropriate machinery, tools and personal protective equipment used for conversion activities. |

# Range of Conditions

Not Applicable

# Unit Mapping Information

FPITMM3201B Convert timber

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPTMM3201 Convert timber

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPITMM3201B Convert timber, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

follow organisational policies and practices relevant to converting timber of at least one of the following materials:

native timber species

imported timber species

dressed timber

in-the-rough timber

green timber

stress and non-stress graded timber

preservative treated timber

coated and/or treated timber products

beams or laminated beams

cut material to width and thickness in line with work order and within prescribed organisational tolerances

conduct operator maintenance on an automatic feed controlled rip saw

stack batches of cut material in line with organizational requirements and OHS regulations

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

timber conversion processes including:

re-sawing in-the-rough boards to optimize single boards through cutting away waste edges

cutting boards into two or more boards thereby maximizing the amount of product in line with the work order

saw set up including setting depth, width and thickness of cut measured from the fence in line with product order and allowing for kerf (width of saw cut)

storage layout and categories for:

raw materials

stacking finished materials, including

preparing for transport

categorizing in common size, length, width and thickness

marking lots in line with work order and site requirements

locating so as not to block access or passage

characteristics of timber and timber defects including:

warp

wane

cupping

shakes

insect defects

knots

resin pockets

cutting patterns and sequences

tolerances

saw feed rates

industry standard cross-sections and lengths

types of conversion tools and equipment; their purpose, features, operation and basic non-specialist repair and maintenance:

measuring equipment

automatic feed controlled ripping saws

saws with adjustable thickness (sloping table)

saws with one blade

rip saws

table saws

bench saws

methods for assessing saw blade condition

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning of plant, tools and equipment

disposing of, recycling and reusing timber and other waste

organisational procedures specific to conversion activities:

workplace health and safety with particular emphasis on equipment lock out

use and maintenance of equipment

communication reporting lines

recording and reporting cutting process and equipment faults.

# Assessment Conditions

The following resources must be made available:

one or more of the following items of conversion equipment:

automatic feed controlled ripping saws

saws with adjustable thickness (sloping table)

saws with one blade

rip saws

table saws

bench saws

measuring equipment

maintenance tools and equipment for saws

consumable components; saw blades

a diverse, comprehensive and commercial product range of timber

personal protective equipment suitable for conversion activities

manufacturers’ instructions for use and maintenance of equipment

work order for conversion activities with specific instructions for the cutting and storage of timber and timber products

template reports for recording processing and equipment faults

relevant personnel for the purposes of communicating information

organisational procedures for conversion activities.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

#### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47