# Assessment Requirements for FWPTMM3209 Install prefabricated timber building systems on-site

Release: 1

Assessment Requirements for FWPTMM3209 Install prefabricated timber building systems on-site

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 4.0. |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit. There must be evidence that, on at least three occasions, the individual has installed prefabricated timber building systems on-site, for two or more panelised or modular prefabricated timber building system types, including:

cross-laminated timber floor and wall systems

post and beam systems (Glulam, laminated veneer lumber)

panelised floor cassette systems

panelised wall systems (including cladding, insulation, windows or doors)

panelised and pre-finished wall systems (inclining above, electrical, mechanical, plumbing or lining)

pre-finished and fully finished timber-based modules.

In doing the above, the individual has:

complied with health and safety requirements and regulations applicable to workplace operations

identified design requirements and installation sequence of prefabricated timber building systems

identified and selected correct fittings, connections, tie-downs and braces for the installation of prefabricated timber building systems

checked suitability of building structure, slab or floor area for the installation of prefabricated timber building systems

applied workplace quality procedures and processes in the context of installing prefabricated timber building systems

selected and used processes, tools and equipment for measuring, marking out, levelling and securing prefabricated timber building systems into place

placed prefabricated timber building system safely, efficiently and to required tolerances and installed connectors to required specifications

checked that walls and corners are accurately levelled, plumbed and square

identified typical faults and problems and rectified faults

communicated efficiently with others, exchanging feedback and timeline reviews.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

relevant current health and safety legislative obligations and workplace procedures as described in industry health and safety guides and site workplace health and safety plans

hazards associated with the construction work and as identified through safe work method statements (SWMS), risk assessments, manufacturer technical information, regulations and industry health and safety guides, including but not limited to:

building systems and components

methods of work

waste material

risk of fire

power lines

cabling and plumbing

drains covered correctly

access and egress

safety measures and equipment:

fire prevention and emergency procedures

evacuation procedure

signs and barricades

personal protective equipment (PPE)

fall prevention and fall arrest equipment

other methods for reducing the risk from fire and other hazards on a construction site as defined in the National Construction Code and site safety plan

types of task-specific documents, their source, information included and how they are read and interpreted with reference to:

drawings

design specifications

on-site workflow, schedules and targets

method statements

risk assessments

progress charts and timetables

manufacturer information

product certifications

building regulations

basic design and offsite manufacture processes for prefabricated timber building systems for recognising problems that arise from missing information, damaged or faulty prefabricated timber building systems and understanding methods to resolve them

installation design specifications for different prefabricated timber building systems with reference to:

installation sequence

lifting equipment, support connections and techniques

bracing techniques

technical properties and specific requirements of materials, elements, components, insulations and connections

mechanical fixings, tie-downs and braces, connection strengths and installation tolerances

building checks before prefabricated timber building system installation and the importance of checking

testing and checking of prefabricated timber building systems for quality assurance:

required tests for different prefabricated timber building system and methods to undertake them

test sampling and frequency of testing within a managed process

specialised tools and equipment for installation and fixing of different prefabricated timber building systems and methods of operation

methods for measuring and calculating the quantity, length, area, centre of gravity and mass of different prefabricated timber building systems for lifting and installation operations

techniques for accurate placement of prefabricated timber building systems before the full installation

problems or faults related to prefabricated timber systems, including materials, components, connections or insulations that arise from on-site installation and impact on structural, fire and acoustic performance of the system

critical performance metrics used in the on-site installation operations of prefabricated timber building systems regarding quality, cycle time, productivity and inventory

methods for reporting:

circumstances that affect the work cycle time

quality checks

product changes or variations during installation

maintenance and servicing requirements for tools and equipment as outlined in workplace procedures or manufacturer manual

waste management procedures regarding safe disposal and recycling.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a construction site or environment that accurately represents workplace conditions

resources, equipment and materials:

specialised tools and equipment for installation and fixing of prefabricated timber building systems

prefabricated timber building systems, components, sections and fittings

PPE, signs and barricades, fall prevention and fall arrest equipment used in building construction operations

•	specifications:

access to workplace health and safety policies, SWMS, emergency procedures, fire safety plans, drawings, design specifications, on-site workflow, schedules and targets, work method statements, risk assessments, manufacturer information and operating instructions for specialised tools and equipment, product certifications and building regulations

relationships:

team member(s) including licenced crane operator and licenced dogger to lift prefabricated timber building systems

timeframes:

according to time specified in job sheet.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47