# FWPTMM3217 Develop knowledge of plywood or laminated veneer lumber production

Release: 1

FWPTMM3217 Develop knowledge of plywood or laminated veneer lumber production

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Application

This unit of competency describes the skills and knowledge required to develop a current understanding of the application and production processes of plywood or laminated veneer lumber (LVL) and identify adhesives and adhesive application systems used in plywood or LVL production.

The unit applies to individuals who are engaged in a range of operational work roles in plywood or LVL production.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Timber Manufactured Products (TMM)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Identify properties and uses of plywood or LVL | 1.1 Identify Australian Standards relevant to LVL or plywood production 1.2 Distinguish between the types of LVL or structural and non-structural plywood products 1.3 Identify timber species used in LVL or structural and non-structural plywood production 1.4 Assess physical and mechanical properties of plywood or LVL 1.5 Identify end-applications of LVL or plywood in building and construction |
| 2. Research plywood or LVL production process | 2.1 Identify characteristics of logs suitable for use in plywood or LVL production 2.2 Identify legislation, standards and codes of practice relevant to plywood or LVL production 2.3 Assess safety and environmental hazards associated with plywood or LVL production 2.4 Determine workflow in plywood or LVL production process and assess factors that may impact on that workflow 2.5 Identify manufacturing processes involved in plywood or LVL production 2.6 Identify quality checks conducted during plywood or LVL production |
| 3. Identify adhesives and adhesive application systems used in plywood or LVL production | 3.1 Identify types, properties, uses and hazards associated with adhesives used in plywood or LVL production as specified in relevant Australian Standards 3.2 Identify factors that impact on the bonding of wood with adhesives used in plywood or LVL production 3.3 Distinguish between thermoset and thermoplastic adhesives 3.4 Identify processes and quality checks that apply in adhesive preparation 3.5 Identify types, uses and advantages and disadvantages of adhesive application systems used in plywood or LVL production |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret safety data sheets on adhesives used in plywood or LVL production Recognise key information in technical documents related plywood or LVL production |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPTMM3217 Develop knowledge of plywood or laminated veneer lumber production | FWPWPP3201 Produce veneer from debarked logs | The unit has been redeveloped to address a skill or task required by industry that is not covered by an existing unit | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPTMM3217 Develop knowledge of plywood or laminated veneer lumber production

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has researched either plywood or laminated veneer lumber (LVL) production and documented:

legislation, standards and codes of practice relevant to plywood or LVL production

timber species and characteristics of logs used in plywood or LVL production

two different end applications of either plywood or LVL products

workflow in plywood or LVL production

production processes for either plywood or LVL production

one factor that may impact on quality and workflow in plywood or LVL production

properties and one potential hazard of one adhesive product used in plywood or LVL production

one type of adhesive application system used in plywood or LVL production.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

types and uses of plywood or LVL, including:

structural plywood

concrete formwork plywood

marine plywood

non-structural interior and exterior plywood

key requirements of legislation, standards and codes of practice relevant to plywood or LVL production

timber species used in plywood or LVL production, including:

softwood – radiata pine, hoop pine, slash pine

hardwood – eucalypt

physical and mechanical properties of LVL or plywood, including:

cross lamination

dimensional stability

thermal properties

acoustic properties

electrical properties

chemical resistance

workability and bending ratio

plywood density

end applications of plywood or LVL in building and construction, including:

plywood – panel product

LVL – framing member

characteristics of logs from suitable timber species selected for use in plywood or LVL production, including:

size

straightness

nature and quantity of defects

plywood or LVL production process, including:

log storage

log debarking and bucking

conditioning

peeling

drying

grading

layup and bonding

pressing

sanding, trimming and branding

main differences in the manufacturing process for plywood and LVL, including:

grading stage

lay-up stage

pressing stage

quality checks conducted during plywood or LVL production

factors impacting on quality and workflow in plywood or LVL production, including:

availability of suitable timber species and quality

veneer quality

adhesives

bonding

operational, supervisory and management skills

access to and maintenance of plant and equipment

safety and environmental hazards associated with plywood or LVL production

types, properties, uses and hazards of adhesives specified in Australian Standards for plywood or LVL production, including:

Type A bonds – produced from phenol formaldehyde (PF) resin

Type B bonds – produced from melamine fortified urea formaldehyde resin (MUF)

Type C and D bonds – produced from urea formaldehyde resin (UF)

factors that impact on the bonding of wood with adhesives used in plywood or LVL production

types and uses of adhesive application systems, including:

hard rolls

air curtain

spray

extrusion.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

resources, equipment and materials:

reference materials on plywood or LVL production processes and adhesives and adhesive application systems used in plywood or LVL production

internet access and computer software and hardware for accessing technical information on plywood or LVL production

specifications:

Australian Standards relevant to plywood or LVL production.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47