# FWPTMM4213 Test finger jointed timber

Release: 1

FWPTMM4213 Test finger jointed timber

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Application

This unit of competency describes the skills and knowledge required to collect, prepare and test samples of finger jointed timber. The unit includes interpreting test results for the identification of problems in production and making necessary changes to processes.

The unit applies to individuals who are engaged in finger jointed timber production.

All work must be carried out to comply with workplace procedures, according to state/territory health and safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Timber Manufactured Products (TMM)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for testing finger jointed timber | 1.1 Determine job requirements from work order or instruction and, where required, seek clarification from appropriate personnel 1.2 Confirm workplace health and safety and environmental protection practices and/or procedures for task, including use of personal protective equipment 1.3 Confirm legislation, standards and codes of practice relevant to testing finger jointed timber 1.4 Identify and assess hazards and take actions to mitigate risks related to testing operations 1.5 Consult appropriate personnel to ensure that work is coordinated effectively with others in the workplace 1.6 Select tools and equipment for testing operations, and check for serviceability according to workplace procedures 1.7 Collect, label and prepare samples for testing according to workplace procedures |
| 2. Calibrate equipment for testing finger jointed timber | 2.1 Set up equipment for testing strength of finger jointed timber 2.2 Calibrate test equipment according to manufacturer instructions and workplace procedures 2.3 Record calibration results according to workplace procedures |
| 3. Conduct test on finger jointed timber and analyse test results | 3.1 Test sample according to workplace procedures 3.2 Collate information on test results 3.3 Analyse and record test results 3.4 Store samples according to workplace procedures |
| 4. Finalise test on finger jointed timber | 4.1 Communicate test results according to workplace procedures 4.2 Report sub-standard product test results, assist with identification of production problems and recommend corrective actions 4.3 Modify test sampling process in response to identified testing problems and production changes 4.4 Record and report testing outcomes |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret routine information from industry guides and standard references on testing techniques for finger jointed timber |
| Writing | Complete labels legibly and accurately with correct details of sample |
| Oral communication | Ask open and closed questions and use active listening techniques to clarify contents of work orders Provide oral reports on testing activities |
| Numeracy | Measure quantities and dimensions of samples Perform routine calculations and interpret numerical results of testing procedures |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FWPTMM4213 Test finger jointed timber | FWPCOT3208 Test strength of joints | Unit redeveloped to address a relevant skill or task that is required by industry | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47

Assessment Requirements for FWPTMM4213 Test finger jointed timber

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 8.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has tested two samples of finger jointed timber for bond strength and adhesive durability and analysed the test results.

In performing this task, the individual has:

complied with workplace health and safety and environmental protection practices and/or procedures, including use of personal protective equipment (PPE) and safe disposal of waste materials

complied with legislation, standards and codes of practice relevant to testing finger jointed timber

selected and calibrated test equipment for testing operations

collected, labelled and prepared samples

conducted the tests

recorded and reported test results and analysed test results to assist with the identification of production problems

recommended corrective actions, as required.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

key requirements of legislation, standards and codes of practice relevant to testing finger jointed timber

types, purpose and procedure for finger jointed timber tests, including:

bond strength

adhesive durability

finger jointing requirements for finger jointed timber product and their justification, including:

shook requirements – moisture content, straight edges, square freshly cut ends, straight grain at join

joint appearance

glue bond durability, including influencing factors – adhesive storage, adhesive mixing, adhesive cure

equipment for testing finger jointed timber product

techniques and reasons for calibrating testing equipment for finger jointed timber product

hazards associated with selection, labelling and physical testing of finger jointed product, including:

moving equipment

flying debris

rotating saw blades

failures in finger jointed timber characteristics that are determined by test results, including:

glue failure

shallow wood fibre failure

wood fibre failure

breaking load (Modulus of Elasticity; Modulus of Rupture)

problems associated with testing process, including:

calibration of test equipment

test accuracy

sample quality

faulty material

defects

•	workplace procedures specific to sampling and testing finger jointed timber product, including:

health and safety, including isolation procedures, lock-outs, emergency stops, machine guarding and wearing of appropriate PPE

environmental protection, including disposing, recycling and reusing timber and other waste

use of testing equipment

sampling and testing processes

communication reporting lines

recording and reporting on test process and test results.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a timber product manufacturing plant or an environment that accurately represents workplace conditions

resources, equipment and materials:

finger jointed timber

test equipment

PPE required for testing finger jointed timber

specifications:

workplace health and safety and environmental protection policies and/or procedures applicable to finger jointed timber testing operations

workplace procedures for finger jointed timber testing operations

workplace procedures for calibration of testing equipment.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47