# Assessment Requirements for FWPWPP2202 Surface treat raw board

Release: 1

Assessment Requirements for FWPWPP2202 Surface treat raw board

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP2202B Surface treat raw board, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

surface treat raw boards in line with individual work orders for each job; types of board to treat includes:

medium density fibreboard

core for laminated veneer

chipboard

fibreboard

plywood

at the completion of each of the above jobs:

shut-down and clean equipment in line with organisational safety procedures

dispose of excess chemicals in line with environmental protection practices

safely store unused chemicals

stack finished boards in storage location according to common size, thickness, length and surface treatment.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for surface treating raw boards:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing timber

disposing of hazardous substances

storage layout and categories for:

raw materials

stacking finished boards

characteristics of timber panels and the effects of chemical treatment including:

medium density fibreboard

core for laminated veneer

chipboard

fibreboard

plywood

properties, at a basic level of understanding, of chemicals used for surface treating raw boards and their purpose

methods used to safely store chemicals

typical work flow processes for surface treating raw boards

purpose, features, operation, shut down and cleaning of compressor driven spray boom equipment

range of appropriate chemical flow rates to avoid equipment blockages and optimise quality and quantity of production output

organisational procedures specific to surface treating raw boards:

workplace health and safety with particular emphasis on equipment lock out, use of personal protective equipment and handling hazardous substances

communication reporting lines

recording and reporting production outcomes and equipment faults.

# Assessment Conditions

The following resources must be made available:

compressor driven spray boom equipment

personal protective equipment suitable for surface treating raw boards

raw boards which includes one or more of the following:

medium density fibreboard

core for laminated veneer

chipboard

fibreboard

plywood

surface treatment chemicals

manufacturers’:

safety data sheets for surface treatment chemicals

instructions for equipment use

recommendations for cleaning equipment

work order with specific instructions for chemical treatment of timber panel products

template documents for recording production outcomes and equipment faults

relevant personnel for the purposes of communicating information

organisational procedures for surface treating raw boards.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47