# Assessment Requirements for FWPWPP2207 Scarf edges of veneer

Release: 1

Assessment Requirements for FWPWPP2207 Scarf edges of veneer

# Modification History

| Release | Comment |
| --- | --- |
| 1 | Replaces equivalent unit FPIWPP2207B Scarf edges of veneer, which was first released with FPI11 Forest and Forest Products Training Package Version 2.2. This is the first release of this unit in the new standards format. Assessment requirements now specify the performance and knowledge evidence, as well as assessment conditions for this unit of competency. These sections simplify and replace components used in the previous unit format, including Required Skills and Knowledge, Evidence Guide and Range Statement |

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit.  If a specific volume or frequency is not stated below, then evidence must be provided that the following requirements have been performed on at least one occasion:

scarf veneer in line with individual work orders that prescribe different lengths, widths, thicknesses and quantities for each job.

for each of the above jobs, visually inspect veneer to re-classify its quality and use after scarfing.

# Knowledge Evidence

A person competent in this unit must be able to demonstrate knowledge of:

environmental protection practices for forest products factory settings:

reducing water and energy use

cleaning plant, tools and equipment

disposing of, recycling and reusing veneer and other waste

typical flow of material supply in and out of the veneer scarfing area

typical work flow processes and veneer scarfing techniques for optimising sheet sizes

appropriate range of feed rates to produce specified joins and quality finish

methods used to visually assess condition of veneer for strength and typical production defects:

overlaps

gaps

veneer faults

joining spacing’s

key features of industry standard grades for re-classifying quality of scarfed veneer and applicable terminology

purpose, features and operation of equipment used to scarf edges of veneer:

specialised scarfing machines

lifting equipment

methods for assessing cutter and blade condition and completing basic non-specialist repair and maintenance for scarfing machines

organisational procedures specific to scarfing edges of veneer:

workplace health and safety with particular emphasis on equipment lock out and use of personal protective equipment

communication reporting lines

recording and reporting production outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

The following resources must be made available:

specialised scarfing machine

continuous conveyor belt system

lifting equipment

maintenance tools and equipment

consumable cutters and blades

personal protective equipment suitable for scarfing veneer

commercial veneer

manufacturers’ instructions for use and maintenance of equipment

work order for with specific instructions for the scarfing of veneer

template documents for recording production outcomes, equipment faults and maintenance requirements

relevant personnel for the purposes of communicating information

organisational procedures for scarfing veneer.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting.

### Assessor requirements

Assessors must:

hold the appropriate assessor competency standards as outlined in regulations; and

be able to demonstrate vocational competencies at least to the level being assessed; and

be able to demonstrate how they are continuing to develop their VET knowledge and skills as well as maintaining their industry currency and assessor competence.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47