# Assessment Requirements for FWPWPP2213 Cut panels

Release: 1

Assessment Requirements for FWPWPP2213 Cut panels

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FWP Forest and Wood Products Training Package Version 6.0 |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has:

cut four different sets of panels to length and width within the specified tolerances, from at least two of the following materials:

medium density fibreboard

laminated veneer

chipboard

fibreboard

plywood

continuous ply

laminated timber

completed cutting patterns and sequences for single straight cuts and angles requiring one cutting pass set for multiple cuts of the same size

stacked cut panels in the designated storage location according to common length and width

completed operator maintenance on the saw used.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

environmental requirements for disposing of, recycling and reusing timber and other waste

storage layout and categories for:

raw materials

stacking finished panels

typical timber panel defects and how they affect ability to produce quality sawn panels:

warp

cupping

voids

blow out

delamination

cutting patterns and sequences for single straight cuts and angles requiring one cutting pass

range of industry standard lengths, cross-sections and tolerances and applicable terminology

purpose, features and operation of equipment used to cut panels:

measuring equipment

manually controlled docking and trimming saws

band saws

bench saws

panel saws

saws with adjustable angle

saws with one or multiple blades

table saws

methods for assessing saw blade condition and completing basic non-specialist repair and maintenance for saws

workplace safety requirements for cutting panels with a power-driven saw:

hazard identification and control measures

equipment lock-out

personal protective equipment (PPE)

hazardous manual tasks

workplace requirements for recording and reporting production outcomes, equipment faults and maintenance requirements.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

skills must be demonstrated in a wood product manufacturing setting or an environment that accurately represents workplace conditions

resources, equipment and materials:

tools, equipment and machinery to cut panels

range of panel types for cutting

PPE

specifications:

access to work order or instructions detailing the task to be carried by operator

access to workplace safety and environmental policies and procedures applicable to cutting panels using a power-driven saw access to workplace procedures and forms for recording production, quality and maintenance reports

manufacturer specifications for power-driven saw used to cut panels

relationships:

person to confirm work order and instructions and report production issues and equipment faults.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=0d96fe23-5747-4c01-9d6f-3509ff8d3d47